# Minidoka Vegetation Sampling Geodatabase User Guide

Table of Contents

Introduction	2

Data Model Description	4

Procedures for Checking Out and Checking in ArcPad Layers for Field Data Collection	6

Prepare files for check out to mobile device	8

Prepare Reference Layers	13

Definition Queries	14

Load the data to the GPS Unit	16

ArcPad Instructions	19

Setting up the GPS Maps for Field Work	19

Background Layers	20

Editing Features	21

All Form Properties	23

Sample ID - Group Form*	24

Alliance_Assoc_EIA Form*	25

Alliance Data Entry Form	26

Map Class Data Entry Form	27

Association Data Entry Form	27

EIA1 Data Entry Form	28

EIA2 Data Entry Form	28

Site Info Data Entry Form	29

Comments Data Entry Form	29

Species Form	32

Species Data Entry Form	32

Comments Entry Form	33

ArcPad Navigation Aids	33

Full Segmentation Layer	33

Labelling Features within ArcPad	33

Go To Navigation	36

GPS Unit Performance	38

Check-in Data from GPS to Geodatabase (ArcMap)	40

Data Definition Tables	45

Table 1. Alliance Codes	46

Table 2. Alliance Scientific Names	47

Table 3. Alliance Common Names	49

Table 4. Association Code	51

Table 5. Association Name	53

Table 6. Association Shortened Scientific Name	55

Table 7. Group Code	61

Table 8. Group Name	62

Table 9. MacroGroup Code	63

Table 10. MacroGroup Name	63

Table 11. Species Common Name	64

Table 12. Species Scientific Name	73

Table 13. EIA Scores	81

Table 14. Cover Classes	81

Table 15. Height Classes	82

Table 16.  Alliance Map Labels	82

Table 17. Alliance Aggregate Classes	83

Versioning	84

# Introduction

The Minidoka Vegetation Sampling Geodatabase was developed in 2015 to support a field data collection effort to expand baseline knowledge of the vegetation communities and their condition (with comparison to high quality reference sites in the region [EIA]).  Refuge staffs supported by a team of botanical experts on contract through NatureServe and the State of Idaho will collect vegetation community information along with a set of other habitat variables and measures. This information will inform a vegetation mapping/modeling process to generate a current cover map.

The field teams will be equipped with Trimble Juno3B GPS units running ArcPad.

The Minidoka Vegetation Sampling Geodatabase is a relational geodatabase consisting of one vector (point) feature class. Linked to the feature class (to be referred to as the Veg_Points for the remaining document), are two relational tables; Alliance_Assoc_EIA and Species

By developing this as a relational model it is possible to repeat monitoring for a point feature over a long period of time in a tidy data model that will be readily available for analysis.

Veg_Point (vector) Feature Class Primary Key: Sample_ID

Alliance_Assoc_EIA

Species

This document will describe the data model and provide a process for using this data model along with a custom designed ArcPad form (.apl) for field data collection.

# Data Model Description

This geodatabase and procedure works best when running ArcGIS 10.3.1 and ArcPad 10.2. R3.

Computer must have ArcPad 10 or higher and Microsoft Mobile Device Center installed.

Vegetation survey data will be stored in a point feature class. Two related tables store the specific information related to the vegetation class and Environmental Integrity Assessment (EIA).

- Veg_Points

- Alliance_Assoc_EIA (Table 1)- Alliance, Map Class, Association, EIA metrics and comments.

- Species (Table 2)- Species

A segmented layer was generated using eCognition, this segmentation delineates similar habitat into vegetation community polygons. The vegetation survey data will  be used to train a classifier and label the polygons into NVCS classes.

- Sample_Polygons

Sample sites were randomly selected using the segmentation layer. Samples were spatially balanced across the refuge and stratified by landcover types (NLCD). Segments are classed if they are selected for sampling.

- Sample_Polygons (Definition Query Selected_Sample = 1)

Management Units were creatd and are included in the geodatabase for planning and navigation purposes. The outer boundaries of the management units also represent the boundaries of the refuge.

- Management_Units

In 2016, a draft map was created using 2015 field data as training points. This map utilizes the 2015 segmentation layer and has labelled the polygons with Alliance Map Labels (these are aggregated Alliances for the most part), Ruderal estimates, other structure attributes.

- Draft_VegMap_MIN_2016

Minidoka collects and manage their data in their local UTM projection, so to facilitate the use of this system with their already existing data (such as roads, management units, aerial imagery, etc.) the preferred projection is local UTM zone.

Minidoka_Veg_2016_RGDB_v1.gdb

Minidoka_Veg_2016_RGDB_v1.mxd

Layer files- symbology  (ArcMap and ArcPad) are available for the feature classes included in the geodatabase.

- Draft_VegMap_MIN_2016.lyr

- Sample_Polygons.lyr (Definition Query set on)

- Sample_Points.lyr (Derived center points for the sample Polygons, may be less intense on the units processing to navigate using the point layer, then on approach turn on the segmentation)

- Full_Segments.lyr (Sample_Polygons)

- Veg_Points.lyr

- Management_Units.lyr

A custom layer file for the Veg_Points feature class to be used on the GPS unit, which will generate a custom field form is provided.

Minidoka_Veg_2016_RGDB_v1.apl

# Procedures for Checking Out and Checking in ArcPad Layers for Field Data Collection

A preloaded MXD is provided. Fix broken links as needed.

If no MXD is available, Start ArcGIS, add the data layer from MMG_GCS_v1.gdb* > Grid_50m_GCS

Zoom to your refuge, Save Map Document.

If needed, Use the Layer Properties>Symbology to set the layer properties. Layers are provided for the following feature classes.

- Draft_VegMap_MIN_2016.lyr

- Sample_Polygons.lyr (Definition Query set on)

- Sample_Points.lyr (Derived center points for the sample Polygons, may be less intense on the units processing to navigate using the point layer, then on approach turn on the segmentation)

- Full_Segments.lyr (Sample_Polygons)

- Veg_Points.lyr

- Management_Units.lyr

Ensure ArcPad tools are properly setup in ArcGIS.

Go to Customize and turn on ArcPad Data Manager.

Next, go to Toolbars and turn on ArcPad Data Manager.

## Prepare files for check out to mobile device

First set the viewer to the extent of the data you want to check-out. Smaller extent checkouts can be preferable if you intend to package up imagery to check out in your project or if you have a large refuge or survey area. It is recommended if you have a large survey area to break your checkout into smaller ‘units’. Large extents will take longer to process and can slow down the GPS unit response times.

The recommended method is to perform a checkout based on the Management Unit extent if you at a large refuge.

Preprocessing Unit checkouts for field work can be a time saver, by doing this in advance you can be at the ready to visit any part of the refuge on short notice.

Go to the ArcPad Data Manager Toolbar

Click Get Data for ArcPad. Next to Welcome Screen (you can select the box to Not Show Again).

The Select Data screen will open. Update the Action column (right click) for each of the layers. Many Some layers can be checked out as background layers for navigation or other orientation purposes, but they all must have the same projection as the Veg_Points grid feature class. ArcPad cannot project on the fly.

ArcPad provides very simple Go To functionality for editable layers. Recommend to Checkout Data for editing for navigation as well as data collection where it makes sense Veg_Points, Selected Sample Segments.

But we will be updating the Vegetation Points (Veg_Points) file. Update the Action for the point layer, Check Out for disconnected editing in ArcPad > Data based on defined extent. The defined extent is that extent shown in the map viewer/data frame window.

Next, check the check box at the end of the row to use the custom form called Minidoka_Veg_2016_RGDB_v1.apl

Checking the box will display the Open Layer ‘explorer’.

Navigate to the location of the Minidoka_Veg_2016_RGDB_v1.apl and click Open.

Click Next to Select the Picture Options. Update the folder path when the Photo field is being used in the data model, Click Next.

Click Next to Select the Output Options.

Use the current display extent to focus your data collection on the area to be surveyed using this checkout. It is recommended to only check out as large an extent as needed. Use of predefined bookmarks and survey target extents is recommended.

Create a folder to store the data. Set where you want to store the data. It is best to set-up a directory for this specifically. For example, ‘Veg_Checkouts’.

When the data from ArcMap is checked out a folder will be created to hold the checkout.

Tip: Name this folder will descriptive elements such as When_What_Where_Who. An example might be ‘20150531_Veg_Unit1_ers’.

Tip: Notice the date format, it is the best format for organizing your files according to date.

Lastly, give the map that will be opened in ArcMap a name. An easy way to keep it all straight is to use the folder name, you can just copy and paste. Click Next.

Leave all the other settings with the defaults on the Deployment Options screen.

Click Finish.

This could take some time depending on the extent of the checkout. Be patient!

You ought to receive a message. Review to make sure the number of feature layers matches what you would expect.

Click OK to close dialog box.

Tip: The symbology that is set in the current ArcMap document will be used in ArcPad.

### Prepare Reference Layers

A good strategy for reference layers is to check them out and put them in a general file folder on the GPS unit, they can be added to a project at any time but do not need to be checked out every time. This reduces the checkout time and ensures the necessary files are always available on the GPS unit when in the field.

Preprocessed Checkouts are available with layer symbology. See ArcPad Instructions for more information on how to add these layers to your check-out.

- VegMap_Background

- Full_ Segmentation

The VegMap layer is simply too big for the Juno to handle. It is essential to narrow down the extent of the checkout for field use.

### Definition Queries

Definition queries are a nice way to limit the data you take into the field. For example, the sample frame (Selected Sample Segments) is coded with Management Units, each of these units have survey sites. You can limit the checkout procedure to only checkout a specific management unit.

## Load the data to the GPS Unit

Open Microsoft Mobile Device Manager

Plug your Trimble into the computer using the USB cord.

You will see a couple messages on the GPS indicating it is communicating and Microsoft Mobile Device Center should eventually indicate Connected. Connect without setting up your device.

Go to Connect without setting up your device, click.

Go to File Management. Browse the contents of your mobile device.

That will bring up an explorer window with your device showing. Double click on the drive.

Go to My Documents and create a folder for the project on the Trimble unit. In the example below the folder is named ‘Veg_Checkouts’. This folder will be used to store all your checkouts.

Now, go back to the location where you saved your ArcPad data and Copy the entire folder of prepared files. And any Background layer checkout you need to have.

Return to the Trimble folder (set up in the previous step). And paste into the working directory folder. By copying this folder over, you have moved a copy to the GPS which will be used in the field (edits and new data collection).

It is always good practice to open the map on the unit and take a test point before heading out to the field. You want to make sure everything loaded properly before you are out in the middle of nowhere!

If more than one field crew or field tech is going to collect data, do a separate checkout for each and name the folders accordingly.

# ArcPad Instructions

## Setting up the GPS Maps for Field Work

These steps can be done in the office prior to going out in the field.

Go to the GPS unit, Click the ArcPad icon and Choose map to open.

Navigate to the folder where the .apm (ArcPad map document) is stored. Open folder select the .apm file. Select the .apm and click Open.

Once the map opens zoom into a scale you can clearly see the sample points.

If you are connected to satellites you can alternatively set the GPS to Center on GPS. This will put the crosshairs in the middle of the screen and show you your position. When walking a red arrow like feature will point in the direction you are moving. This red arrow feature disappears when standing still.

## Background Layers

If you are using pre-prepared background layers, add them into the project at this point.

Since you do not want reference layers on at all times, go to the Layer properties and turn off the visibility.

Save the .apm file.

Repeat the process for other background layers.

The VegMap layer is simply too big for the Juno to handle. It is essential to narrow down the extent of the checkout for field use.

## Editing Features

When it is time to collect attributes, turn on editing for the point layer. Notice the red box around the icon, that red box indicates it is editable.

Practice taking a point using GPS capture and taking a point using the select tool.

To create a point, select the blue arrow and click on the map, OR use Select at GPS Position (if you are standing in the segment).

Once the feature is placed on the map  the custom form should appear.

If you do not see the above form you did not check out the data using the custom form. Redo the process to ensure you have full functionality during your data collection.

## All Form Properties

The following sections describe the requested data fields in each geodatabase form. The highlighted fields are required, the rest are optional but please fill them out if you have the opportunity.

Not all of these forms need to be filled out while in the field. The underlined form names with an asterisks need to be filled out in the field, the rest can be done in the office.

This geodatabase is relational. The Sample ID – Group  is the primary feature class, it represents a layer made up of Veg_Points. Each point is related to two tables. The relationship between the feature class and the related tables is one:many. Meaning, for each point there can be many records associated in each of the tables. Multiple records (or attributes) are captured by filling out the respective form, selecting the blue arrow to get back to the main form page, and then selecting the add data button again (blue circle with white plus sign). Repeat this work flow as many times as you need to enter all appropriate attributes. This process will be described in more detail below.

## Sample ID - Group Form*

This form is directly tied to the Veg_Point Feature Class.

Use the arrows to move between the tabs in order to enter data in the various relational tables. Each table has its own tab.

The graphic below is expanded to show all the tabs, each tab corresponds to the feature class  and the relational tables   .

Work through as many of the tabs as necessary, and then click the green OK button.

The green OK button completes the record (a feature class record and related table records), and closes the forms.

Do not click the green OK button until you have completed the Grid Properties and the Related Table tabs, as needed.

Sample ID: Refer to the Sample Segments or Full Segmentation layer for the Sample ID. Only one point should be placed within any given segment.

MacroGroup Name:

Group Name:

## Alliance_Assoc_EIA Form*

This form is used to report Alliance, Association community classification and EIA scores at the observation site.

Alliance, Association and EIA scores are stored in a relational table. To add to the table select the blue circle with plus sign.

There are a series of forms that you will work through to add all the attributes for the site.

See Table X for list of choices and definitions. (Relational table, multiple records may be entered for a single cell)

When the tab is selected the attribute table is displayed showing all records for the selected feature (grid cell).

Clicking on a record in the attribute table will select it and a record marker will show to the far left of the record and it may be highlighted

The show attributes button  can be used to review the record in form view, or a scrollbar can be used to scroll across and review the attributes.

Click the blue plus sign button   to open the form and enter data.

The red X   will delete the selected record in the attribute table

### Alliance Data Entry Form

Click the blue arrow button  after updating the data fields to close the form and return to the attribute table.

The green arrow is useful if you will be repeating an attribute, such as Observer or Alliance. This will save the value for the next record, and use it as a temporary default.

To turn this on, fill in the attributes you will want repeated and click the green arrow. It will then become highlighted . The default setting can be deactivated on any subsequent new record by re-clicking the green arrow, this will remove the highlighting.

### Map Class Data Entry Form

### Association Data Entry Form

### EIA1 Data Entry Form

### EIA2 Data Entry Form

### Site Info Data Entry Form

This form is used to report other site information for refuge analysis and mapping at the observation site. Values for percent of biologic crust, bare soil, rock, litter and ruderal must be recorded as an integer (no decimals, no special characters, no letter. Values should be within the range of 0-100.

### Comments Data Entry Form

TIP: Alternate Workflow for Alliance_Assoc_EIA Tab

An alternate workflow for collecting the  attributes is to scroll over to the Attributes tab. The Attributes tab lists every field in the forms described above and can be used for data entry. The example below has been expanded to show the entire attribute table. In general only a small portion can be viewed on handheld devices so more scrolling may be required.

Expanded Desktop View

Typical handheld view

## Species Form

This form is used to report known species that occur on the site.

See Table X for list of choices and definitions.  Species are stored in a relational table, multiple records may be entered for a single point. When the tab is selected the attribute table is displayed showing all records for the selected feature. If new feature the table will appear blank.

Attribute Table view

Clicking on a record in the attribute table will select it and a record marker will show to the far left of the record and it may be highlighted .

The show attributes button  can be used to review the record in form view, or a scrollbar can be used to scroll across and review the attributes.

Click the blue plus sign button   to open the form and enter data.

The red X   will delete the selected record in the attribute table.

### Species Data Entry Form

Click the blue arrow button  after updating the data fields to close the form and return to the attribute table.

The green arrow is useful if you will be repeating an attribute, such as Observer and Date. This will save the value for the next record, and use it as a temporary default.

To turn this on, fill in the attributes you will want repeated and click the green arrow. It will then become highlighted . The default setting can be deactivated on any subsequent new record by re-clicking the green arrow, this will remove the highlighting.

### Comments Entry Form

## ArcPad Navigation Aids

### Full Segmentation Layer

These features can be selected from the map viewer and used for assisted navigation. They can also be labelled, on the fly, on the field unit.

### Labelling Features within ArcPad

Layers checked out from ArcMap can be labelled on the fly using ArcPad. This is very useful for reviewing and selecting sample sites for surveying.

Go to the main screen. Select the layers icon. A window will open that can be used to control the functionality of each layer. There are many options here such as limiting which layers can utilize the identify window. Also limiting which layers are editable.

To access the symbology and labelling options, select the layer you want to set labels, then click the little hand over feature button.

A new window will open, select the Labels tab. Change the radial button to Labels using field: and set the dropdown to SAMPLE_ID. Click OK. Adjust the font size by clicking the red A.

### Go To Navigation

Go To Selected Features is a handy way to use the ArcPad map as a way to identifying and navigating to your survey sites. Once you have the labels turned on you can clearly see the plot order of all the available survey sites.

To use this functionality go to the edit menu and turn on the editing for the survey sites layer. To use navigation this way the layer that you want to target must be an editable layer!

Then Select the next site using the select tool. And then click Go to Selected Feature.

The destination is displayed with a Mark label.

Use the GPS Position Window to view the distance (DST) and bearing (BRG) from your current GPS position to your selected destination

Tip

Activating the GPS: It is not necessary to activate the GPS before selecting a navigation destination. However, the GPS does need to be activated in order to open the GPS Position Window and view the distance and bearing to the destination.

Tip

Clearing the selected destination: To clear the selected destination and remove the destination Mark label, tap the Clear Selected Feature located on the drop-down list below the Select menu on the toolbar. The Mark label is displayed when a destination has been selected.

### GPS Unit Performance

GPS unit performance can be improved by reducing the number of visible layers while navigating. It can take a lot of processing power to redraw very detailed layers such as the segmentation or draft veg map. It is best to keep these turned off until they are needed (you are within range of the target).

It is always best practice to stop editing after data collection is finished and before exiting the map, and save and close your map on the GPS unit before downloading the data back to your computer.

*************************************************************************************

Scroll through each of the tabs and fill in the attributes that are appropriate.

Be sure to click the green OK to save.

It is always best practice to stop editing after data collection is finished and before exiting the map, and save and close your map on the GPS unit before downloading the data back to your computer.

# Check-in Data from GPS to Geodatabase (ArcMap)

Connect your GPS to the computer and let Mobile device center connect.

Now the data will be copied from the device to your checkout folder, and you will be replacing the checkout with your edited version. But first, make a backup! Make a copy and stick it into your check-in backup folder before you replace the actual checked out folder.

When you attempt to overwrite the checked out (not edited) folder with your edited version you will receive a warning, click Yes and proceed.

Return to ArcPad Data Manager.

Use the Green plus to navigate to your edited checkout data. Select the axf file. You will then see a summary of how many features and relational table records were added and/or modified. Select the Feature Classes with new or modified data. Click Check in.

If you do this process outside an edit session you will receive a warning that you cannot undo. If you are worried; Cancel, go into an edit session and return to this point that way you can undo if need be.

The check-in process may take some time depending on the number of features collected and the size of the extent. Please be patient!

A Results dialog will appear and look to see that your layers were successfully checked in.

Scroll across and look for the successfully checked in message. Click OK. Then Cancel out of the Get Data from ArcPad window.

Go look at your data!

Remember the data is now normalized across one feature class and two relational tables. The link between the tables is the Sample_ID.

The Identify tool is also useful for looking at records in relational tables.

# Data Definition Tables

### Table 1. Alliance Codes

| AllCode |  |
| --- | --- |
| Code | Description |
| A1046 | A1046 |
| A1262 | A1262 |
| A1270 | A1270 |
| A1329 | A1329 |
| A1332 | A1332 |
| A1374 | A1374 |
| A1814 | A1814 |
| A3183 | A3183 |
| A3194 | A3194 |
| A3195 | A3195 |
| A3196 | A3196 |
| A3202 | A3202 |
| A3257 | A3257 |
| A3496 | A3496 |
| A3497 | A3497 |
| A3566 | A3566 |
| A3759 | A3759 |
| A3798 | A3798 |
| A3799 | A3799 |
| A3800 | A3800 |
| A3806 | A3806 |
| A3807 | A3807 |
| A3847 | A3847 |
| A3848 | A3848 |
| A3849 | A3849 |
| A3895 | A3895 |
| A3896 | A3896 |
| A3927 | A3927 |
| A3975 | A3975 |
| A3985 | A3985 |
| A3988 | A3988 |
| A4011 | A4011 |
| A4148 | A4148 |
| A4149 | A4149 |
| A4165 | A4165 |
| A4192 | A4192 |
| A4213 | A4213 |
| A4217 | A4217 |
| Anew01 | Anew01 |
| Anew02 | Anew02 |
| Anew03 | Anew03 |
| Anew04 | Anew04 |
| nd | No Data |
| sc | See Comments |
| zb | Bare Ground |
| zc | Cultivated Crops |
| zd | Developed |
| zw | Open Water |

### Table 2. Alliance Scientific Names

| AllSciName |  |
| --- | --- |
| Code | Description |
| Achnatherum hymenoides - Pseudoroegneria spicata - Muhlenbergia pungens Grassland Alliance | Achnatherum hymenoides - Pseudoroegneria spicata - Muhlenbergia pungens Grassland Alliance |
| Agropyron cristatum Western Ruderal Perennial Grassland Alliance | Agropyron cristatum Western Ruderal Perennial Grassland Alliance |
| Amaranthus (californica, powellii) Sparce Mud Flat Alliance | Amaranthus (californica, powellii) Sparce Mud Flat Alliance |
| Artemisia spp. - Mixed Shrub Ruderal Understory Shrubland Alliance | Artemisia spp. - Mixed Shrub Ruderal Understory Shrubland Alliance |
| Artemisia tridentata ssp. tridentata - Artemisia tridentata ssp. xericensis Dry Shrubland Alliance | Artemisia tridentata ssp. tridentata - Artemisia tridentata ssp. xericensis Dry Shrubland Alliance |
| Artemisia tridentata ssp. tridentata - Artemisia tridentata ssp. xericensis Mesic Shrubland & Steppe Alliance | Artemisia tridentata ssp. tridentata - Artemisia tridentata ssp. xericensis Mesic Shrubland & Steppe Alliance |
| Bare Ground | Bare Ground |
| Bromus tectorum - Taeniatherum caput-medusae Ruderal Annual Grassland Alliance | Bromus tectorum - Taeniatherum caput-medusae Ruderal Annual Grassland Alliance |
| Carex praegracilis - Carex scopulorum - Eleocharis quinqueflora Herbaceous Alliance | Carex praegracilis - Carex scopulorum - Eleocharis quinqueflora Herbaceous Alliance |
| Centaurea solstitialis - Isatis tinctoria - Salsola tragus Ruderal Annual Forb Alliance | Centaurea solstitialis - Isatis tinctoria - Salsola tragus Ruderal Annual Forb Alliance |
| Chrysothamnus viscidiflorus Shrubland & Shrub Herbaceous Alliance | Chrysothamnus viscidiflorus Shrubland & Shrub Herbaceous Alliance |
| Conyza canadensis - Cirsium arvense - Lactuca serriola Ruderal Herbaceous Alliance | Conyza canadensis - Cirsium arvense - Lactuca serriola Ruderal Herbaceous Alliance |
| Cultural Vegetation | Cultural Vegetation |
| Developed | Developed |
| Distichlis spicata Alkaline Wet Meadow Alliance | Distichlis spicata Alkaline Wet Meadow Alliance |
| Elaeagnus angustifolia Ruderal Woodland Alliance | Elaeagnus angustifolia Ruderal Woodland Alliance |
| Eleocharis palustris - Eleocharis acicularis Marsh Alliance | Eleocharis palustris - Eleocharis acicularis Marsh Alliance |
| Elymus lanceolatus - Hesperostipa comata - Phacelia hastata Central Rocky Mountain Sand Deposit Grassland Alliance | Elymus lanceolatus - Hesperostipa comata - Phacelia hastata Central Rocky Mountain Sand Deposit Grassland Alliance |
| Ericameria nauseosa - Eriogonum leptocladon - Tetradymia tetrameres Sparse Scrub Alliance | Ericameria nauseosa - Eriogonum leptocladon - Tetradymia tetrameres Sparse Scrub Alliance |
| Ericameria nauseosa Shrubland & Shrub Herbaceous Alliance | Ericameria nauseosa Shrubland & Shrub Herbaceous Alliance |
| Festuca idahoensis - Pseudoroegneria spicata - Pascopyrum smithii Mesic Grassland Alliance | Festuca idahoensis - Pseudoroegneria spicata - Pascopyrum smithii Mesic Grassland Alliance |
| Great Basin & Intermountain Ruderal Shrubland | Great Basin & Intermountain Ruderal Shrubland |
| Hesperostipa comata Bunch Herbaceous Alliance | Hesperostipa comata Bunch Herbaceous Alliance |
| Juncus balticus - Juncus mexicanus Herbaceous Alliance | Juncus balticus - Juncus mexicanus Herbaceous Alliance |
| Juniperus osteosperma / Herbaceous Understory Woodland & Savanna Alliance | Juniperus osteosperma / Herbaceous Understory Woodland & Savanna Alliance |
| Juniperus osteosperma / Shrub Understory Woodland Alliance | Juniperus osteosperma / Shrub Understory Woodland Alliance |
| Krascheninnikovia lanata Dwarf-shrubland & Dwarf-shrub Herbaceous Alliance | Krascheninnikovia lanata Dwarf-shrubland & Dwarf-shrub Herbaceous Alliance |
| Leymus cinereus - Leymus triticoides Herbaceous Alliance | Leymus cinereus - Leymus triticoides Herbaceous Alliance |
| No Data | No Data |
| Open Water | Open Water |
| Phragmites australis - Arundo donax - Alopecurus pratensis Ruderal Marsh Alliance | Phragmites australis - Arundo donax - Alopecurus pratensis Ruderal Marsh Alliance |
| Physocarpus malvaceus - Symphoricarpos albus Mesic Shrubland Alliance | Physocarpus malvaceus - Symphoricarpos albus Mesic Shrubland Alliance |
| Poa pratensis - Agrostis gigantea - Agrostis stolonifera Ruderal Marsh Alliance | Poa pratensis - Agrostis gigantea - Agrostis stolonifera Ruderal Marsh Alliance |
| Poa secunda - Muhlenbergia richardsonis - Carex douglasii Moist Meadow Alliance | Poa secunda - Muhlenbergia richardsonis - Carex douglasii Moist Meadow Alliance |
| Populus angustifolia Riparian Woodland & Forest Alliance | Populus angustifolia Riparian Woodland & Forest Alliance |
| Populus deltoides - Populus alba- Robina pseudoacacia Planted/Escaped Woodland | Populus deltoides - Populus alba- Robina pseudoacacia Planted/Escaped Woodland |
| Populus deltoides ssp. wislizeni - Populus deltoides ssp. monilifera - Salix amygdaloides Riparian Woodland Alliance | Populus deltoides ssp. wislizeni - Populus deltoides ssp. monilifera - Salix amygdaloides Riparian Woodland Alliance |
| Potamogeton natans - Polygonum amphibium Aquatic Vegetation Alliance | Potamogeton natans - Polygonum amphibium Aquatic Vegetation Alliance |
| Redfieldia flexuosa - Leymus flavescens - Achnatherum hymenoides Grassland Alliance | Redfieldia flexuosa - Leymus flavescens - Achnatherum hymenoides Grassland Alliance |
| Rhus trilobata - Crataegus rivularis - Forestiera pubescens Shrubland Alliance | Rhus trilobata - Crataegus rivularis - Forestiera pubescens Shrubland Alliance |
| Salix alba - Salix fragilis Ruderal Riparian Forest Alliance | Salix alba - Salix fragilis Ruderal Riparian Forest Alliance |
| Salix exigua - Salix irrorata Shrubland Alliance | Salix exigua - Salix irrorata Shrubland Alliance |
| Salix spp. - Artemisia cana Ruderal Understory Wet Shrubland Alliance | Salix spp. - Artemisia cana Ruderal Understory Wet Shrubland Alliance |
| Sarcobatus vermiculatus Intermountain Shrub Alliance | Sarcobatus vermiculatus Intermountain Shrub Alliance |
| Schoenoplectus americanus - Schoenoplectus acutus - Scirpus microcarpus Herbaceous Emergent Alliance | Schoenoplectus americanus - Schoenoplectus acutus - Scirpus microcarpus Herbaceous Emergent Alliance |
| See Comments | See Comments |
| Suaeda occidentalis - (& other Alkaline annual forbs) Sparse Semi-Alkaline Wet Meadow | Suaeda occidentalis - (& other Alkaline annual forbs) Sparse Semi-Alkaline Wet Meadow |
| Typha domingensis - Typha latifolia - Typha angustifolia Western Herbaceous Emergent Alliance | Typha domingensis - Typha latifolia - Typha angustifolia Western Herbaceous Emergent Alliance |

### Table 3. Alliance Common Names

| AllComName |  |
| --- | --- |
| Code | Description |
| Amaranthus (californica, powellii) Sparce Mud Flat Alliance | Amaranthus (californica, powellii) Sparce Mud Flat Alliance |
| Baltic Rush Wet Meadow Alliance | Baltic Rush Wet Meadow Alliance |
| Bare Ground | Bare Ground |
| Basin Big Sagebrush Dry Shrubland Alliance | Basin Big Sagebrush Dry Shrubland Alliance |
| Basin Big Sagebrush Mesic Steppe & Shrubland Alliance | Basin Big Sagebrush Mesic Steppe & Shrubland Alliance |
| Basin Wildrye - Beardless Wildrye Alkaline Wet Meadow Alliance | Basin Wildrye - Beardless Wildrye Alkaline Wet Meadow Alliance |
| Blowout Grass - Yellow Wildrye - Indian Ricegrass Grassland Alliance | Blowout Grass - Yellow Wildrye - Indian Ricegrass Grassland Alliance |
| Bullrush Marsh Alliance | Bullrush Marsh Alliance |
| Cattail Marsh Alliance | Cattail Marsh Alliance |
| Cheatgrass - Medusa head Ruderal Annual Grassland Alliance | Cheatgrass - Medusa head Ruderal Annual Grassland Alliance |
| Clustered Sedge - Mt Sedge - Fewflower Spikerush Wet Meadow Alliance | Clustered Sedge - Mt Sedge - Fewflower Spikerush Wet Meadow Alliance |
| Common Reed - Giant Reed - Meadow Foxtail Ruderal Marsh Alliance | Common Reed - Giant Reed - Meadow Foxtail Ruderal Marsh Alliance |
| Common Spikerush - Needle Spikerush Marsh Alliance | Common Spikerush - Needle Spikerush Marsh Alliance |
| Coyote Willow - Bluestem Willow Shrubland Alliance | Coyote Willow - Bluestem Willow Shrubland Alliance |
| Crack Willow - White Willow Ruderal Riparian Forest Alliance | Crack Willow - White Willow Ruderal Riparian Forest Alliance |
| Crested Wheatgrass Western Ruderal Perennial Grassland Alliance | Crested Wheatgrass Western Ruderal Perennial Grassland Alliance |
| Cultural Vegetation | Cultural Vegetation |
| Developed | Developed |
| Eastern Cottonwood - White Poplar - Black Locust Planted/Escaped Woodland Alliance | Eastern Cottonwood - White Poplar - Black Locust Planted/Escaped Woodland Alliance |
| Greasewood Intermountian Wet Shrubland Alliance | Greasewood Intermountian Wet Shrubland Alliance |
| Great Basin & Intermountain Ruderal Shrubland | Great Basin & Intermountain Ruderal Shrubland |
| Green Rabbitbrush Shrubland Alliance | Green Rabbitbrush Shrubland Alliance |
| Horseweed - Canada Thistle - Prickly lettuce Ruderal Herbaceous Alliance | Horseweed - Canada Thistle - Prickly lettuce Ruderal Herbaceous Alliance |
| Idaho Fescue - Bluebunch Wheatgrass - Mesic Grassland Alliance | Idaho Fescue - Bluebunch Wheatgrass - Mesic Grassland Alliance |
| Indian Ricegrass - Bluebunch wheatgrass - Sandhill Muhly Grassland Alliance | Indian Ricegrass - Bluebunch wheatgrass - Sandhill Muhly Grassland Alliance |
| Kentucky Bluegrass - Redtop - Creeping Bentgrass Ruderal Marsh Alliance | Kentucky Bluegrass - Redtop - Creeping Bentgrass Ruderal Marsh Alliance |
| Narrowleaf Cottonwood Riparian Woodland Alliance | Narrowleaf Cottonwood Riparian Woodland Alliance |
| Needle and Thread Grassland Alliance | Needle and Thread Grassland Alliance |
| Ninebark - Snowberry Mesic Shrubland Alliance | Ninebark - Snowberry Mesic Shrubland Alliance |
| No Data | No Data |
| Open Water | Open Water |
| Peach Leaf Willow (Rio Grande - Plains Cottonwood) Woodland Alliance | Peach Leaf Willow (Rio Grande - Plains Cottonwood) Woodland Alliance |
| Pondweed Aquatic Vegetation Alliance | Pondweed Aquatic Vegetation Alliance |
| Rubber Rabbitbrush - Sand Buckwheat - Fourpart Horsebrush Sparse Scrub Alliance | Rubber Rabbitbrush - Sand Buckwheat - Fourpart Horsebrush Sparse Scrub Alliance |
| Rubber Rabbitbrush Shrubland Alliance | Rubber Rabbitbrush Shrubland Alliance |
| Russian Olive Ruderal Riparian Scrub Alliance | Russian Olive Ruderal Riparian Scrub Alliance |
| Sagebrush (native) - Mixed shrub (native) Ruderal understory Shrubland Alliance | Sagebrush (native) - Mixed shrub (native) Ruderal understory Shrubland Alliance |
| Saltgrass Alkaline Wet Meadow Alliance | Saltgrass Alkaline Wet Meadow Alliance |
| Sandberg Bluegrass - Mat Muhly - Douglas' Sedge Moist Meadow Alliance | Sandberg Bluegrass - Mat Muhly - Douglas' Sedge Moist Meadow Alliance |
| See Comments | See Comments |
| Suaeda occidentalis - (& other Alkaline annual forbs) Sparse Semi-Alkaline Wet Meadow | Suaeda occidentalis - (& other Alkaline annual forbs) Sparse Semi-Alkaline Wet Meadow |
| Sumac - Hawthorn - Stretchberry Shrubland Alliance | Sumac - Hawthorn - Stretchberry Shrubland Alliance |
| Thickspike wheatgrass - Needle and Thread Grass - Silver Phacelia Grassland Alliance | Thickspike wheatgrass - Needle and Thread Grass - Silver Phacelia Grassland Alliance |
| Utah Juniper / Grass understory Woodland Alliance | Utah Juniper / Grass understory Woodland Alliance |
| Utah Juniper / Shrub understory Woodland Alliance | Utah Juniper / Shrub understory Woodland Alliance |
| Willow (native) - Silver sagebrush Ruderal Understory Wet Shrubland Alliance | Willow (native) - Silver sagebrush Ruderal Understory Wet Shrubland Alliance |
| Winterfat Steppe & Dwarf-shrubland Alliance | Winterfat Steppe & Dwarf-shrubland Alliance |
| Yellow Star-thistle - Dyer's woad - Prickly Russian thistle Ruderal Annual Forb Alliance | Yellow Star-thistle - Dyer's woad - Prickly Russian thistle Ruderal Annual Forb Alliance |

### Table 4. Association Code

| AssCode |  |
| --- | --- |
| Code | Description |
| CEGL000738 | CEGL000738 |
| CEGL000947 | CEGL000947 |
| CEGL001008 | CEGL001008 |
| CEGL001016 | CEGL001016 |
| CEGL001018 | CEGL001018 |
| CEGL001121 | CEGL001121 |
| CEGL001202 | CEGL001202 |
| CEGL001323 | CEGL001323 |
| CEGL001329 | CEGL001329 |
| CEGL001359 | CEGL001359 |
| CEGL001362 | CEGL001362 |
| CEGL001363 | CEGL001363 |
| CEGL001366 | CEGL001366 |
| CEGL001373 | CEGL001373 |
| CEGL001475 | CEGL001475 |
| CEGL001479 | CEGL001479 |
| CEGL001563 | CEGL001563 |
| CEGL001577 | CEGL001577 |
| CEGL001650 | CEGL001650 |
| CEGL001657 | CEGL001657 |
| CEGL001703 | CEGL001703 |
| CEGL001770 | CEGL001770 |
| CEGL001833 | CEGL001833 |
| CEGL001838 | CEGL001838 |
| CEGL001840 | CEGL001840 |
| CEGL001843 | CEGL001843 |
| CEGL002010 | CEGL002010 |
| CEGL002360 | CEGL002360 |
| CEGL002660 | CEGL002660 |
| CEGL002699 | CEGL002699 |
| CEGL002713 | CEGL002713 |
| CEGL002799 | CEGL002799 |
| CEGL002815 | CEGL002815 |
| CEGL002919 | CEGL002919 |
| CEGL002925 | CEGL002925 |
| CEGL002933 | CEGL002933 |
| CEGL002966 | CEGL002966 |
| CEGL003019 | CEGL003019 |
| CEGL004004 | CEGL004004 |
| CEGL005269 | CEGL005269 |
| CEGL005417 | CEGL005417 |
| CEGL005470 | CEGL005470 |
| CEGL005590 | CEGL005590 |
| CEGL005591 | CEGL005591 |
| CEGL005592 | CEGL005592 |
| CEGL005593 | CEGL005593 |
| CEGL005594 | CEGL005594 |
| CEGL005595 | CEGL005595 |
| CEGL005602 | CEGL005602 |
| CEGL005604 | CEGL005604 |
| CEGL005605 | CEGL005605 |
| CEGL005606 | CEGL005606 |
| CEGL005614 | CEGL005614 |
| CEGL005615 | CEGL005615 |
| CEGL005616 | CEGL005616 |
| CEGL005619 | CEGL005619 |
| CEGL005622 | CEGL005622 |
| CEGL005623 | CEGL005623 |
| No Data | No Data |
| Provisional1 | Provisional1 |
| Provisional2 | Provisional2 |
| Provisional3 | Provisional3 |
| Provisional4 | Provisional4 |

### Table 5. Association Name

| AssComName |  |
| --- | --- |
| Code | Descriptio |
| Baltic Rush Wet Meadow | Baltic Rush Wet Meadow |
| Barnyardgrass Ruderal Wet Meadow | Barnyardgrass Ruderal Wet Meadow |
| Basin Big Sagebrush / Basin Wildrye Shrubland | Basin Big Sagebrush / Basin Wildrye Shrubland |
| Basin Big Sagebrush / Bluebunch Wheatgrass Shrub Grassland | Basin Big Sagebrush / Bluebunch Wheatgrass Shrub Grassland |
| Basin Big Sagebrush / Needle and Thread Shrubland | Basin Big Sagebrush / Needle and Thread Shrubland |
| Basin Big Sagebrush / Sandberg Bluegrass Shrubland | Basin Big Sagebrush / Sandberg Bluegrass Shrubland |
| Basin Wildrye Wet Meadow | Basin Wildrye Wet Meadow |
| Big sagebrush - Rubber Rabbitbrush / Cheatgrass Ruderal Shrubland | Big sagebrush - Rubber Rabbitbrush / Cheatgrass Ruderal Shrubland |
| Black Locust Ruderal Woodland | Black Locust Ruderal Woodland |
| Cheatgrass Ruderal Annual Grassland | Cheatgrass Ruderal Annual Grassland |
| Chokecherry / Big Basin Wildrye Shrubland | Chokecherry / Big Basin Wildrye Shrubland |
| Clustered Field Sedge Wet Meadow | Clustered Field Sedge Wet Meadow |
| Common Reed Western Ruderal Wet Meadow | Common Reed Western Ruderal Wet Meadow |
| Common Spikerush Marsh | Common Spikerush Marsh |
| Cosmopolitan Bulrush Herbaceous | Cosmopolitan Bulrush Herbaceous |
| Coyote Willow / Common Reed Ruderal Wet Shrubland | Coyote Willow / Common Reed Ruderal Wet Shrubland |
| Coyote Willow / Mesic Forbs Shrubland | Coyote Willow / Mesic Forbs Shrubland |
| Crack Willow - White Willow Ruderal Woodland | Crack Willow - White Willow Ruderal Woodland |
| Eastern Cottonwood Western Ruderal Woodland | Eastern Cottonwood Western Ruderal Woodland |
| Floating Pondweed Aquatic Vegetation | Floating Pondweed Aquatic Vegetation |
| Golden Current Shrubland | Golden Current Shrubland |
| Greasewood / Baltic Rush Wet Shrubland | Greasewood / Baltic Rush Wet Shrubland |
| Greasewood / Basin Wildrye Wet Shrubland | Greasewood / Basin Wildrye Wet Shrubland |
| Greasewood / Big sagebrush Wet Shrubland | Greasewood / Big sagebrush Wet Shrubland |
| Greasewood / Indian Ricegrass Wet Shrubland | Greasewood / Indian Ricegrass Wet Shrubland |
| Greasewood / Rubber Rabbitbrush Wet Shrubland | Greasewood / Rubber Rabbitbrush Wet Shrubland |
| Greasewood / Saltgrass Shrubland | Greasewood / Saltgrass Shrubland |
| Green Rabbitbrush / Bluebunch Wheatgrass Shrubland | Green Rabbitbrush / Bluebunch Wheatgrass Shrubland |
| Green Rabbitbrush / Cheatgrass Ruderal Shrubland | Green Rabbitbrush / Cheatgrass Ruderal Shrubland |
| Green Rabbitbrush / Crested Wheatgrass Ruderal Shrubland | Green Rabbitbrush / Crested Wheatgrass Ruderal Shrubland |
| Green Rabbitbrush / Kentucky Bluegrass Ruderal Shrubland | Green Rabbitbrush / Kentucky Bluegrass Ruderal Shrubland |
| Green Rabbitbrush / Needle and Thread Shrubland | Green Rabbitbrush / Needle and Thread Shrubland |
| Green Rabbitbrush / Sandburg bluegrass - Cheatgrass Ruderal Shrubland | Green Rabbitbrush / Sandburg bluegrass - Cheatgrass Ruderal Shrubland |
| Green Rabbitbrush / Western Wheatgrass Shrubland | Green Rabbitbrush / Western Wheatgrass Shrubland |
| Hardstem Bulrush Marsh | Hardstem Bulrush Marsh |
| Indian Ricegrass - Lemon Scurfpea Grassland | Indian Ricegrass - Lemon Scurfpea Grassland |
| Lewis Mock Orange / Scabland Penstemon Shrubland | Lewis Mock Orange / Scabland Penstemon Shrubland |
| Narrowleaf Cottonwood / Basin Big Sagebrush - Sumac Riparian Woodland | Narrowleaf Cottonwood / Basin Big Sagebrush - Sumac Riparian Woodland |
| Narrowleaf or Broadleaf Cattail Marsh | Narrowleaf or Broadleaf Cattail Marsh |
| Needle and Thread - Indian Ricegrass Grassland | Needle and Thread - Indian Ricegrass Grassland |
| No Data | No Data |
| Peachleaf Willow Woodland | Peachleaf Willow Woodland |
| Rubber Rabbitbrush / Yellow Wildrye / Lemon Scurfpea Shrubland | Rubber Rabbitbrush / Yellow Wildrye / Lemon Scurfpea Shrubland |
| Rubber Rabbitbrush Shrubland | Rubber Rabbitbrush Shrubland |
| Russian Olive Ruderal Woodland | Russian Olive Ruderal Woodland |
| Russian Thistle Ruderal Grassland | Russian Thistle Ruderal Grassland |
| Saltgrass Wet Meadow | Saltgrass Wet Meadow |
| Sandburg bluegrass - Cheatgrass Ruderal Grassland | Sandburg bluegrass - Cheatgrass Ruderal Grassland |
| Sandburg Bluegrass - Granite Prickly Phlox Moist Meadow | Sandburg Bluegrass - Granite Prickly Phlox Moist Meadow |
| Sandburg Bluegrass Moist Meadow | Sandburg Bluegrass Moist Meadow |
| Seepweed Herbaceous Vegetation | Seepweed Herbaceous Vegetation |
| Skunkbush Sumac Moist-Wet Shrubland | Skunkbush Sumac Moist-Wet Shrubland |
| Tansy Mustard - Cheatgrass Ruderal Grassland | Tansy Mustard - Cheatgrass Ruderal Grassland |
| Thickspike Wheatgrass / Lupine Grassland | Thickspike Wheatgrass / Lupine Grassland |
| Thurber's Needlegrass - Cheatgrass Ruderal Grassland | Thurber's Needlegrass - Cheatgrass Ruderal Grassland |
| Utah Juniper / Basin Big Sagebrush / Bluebunch wheatgrass Woodland | Utah Juniper / Basin Big Sagebrush / Bluebunch wheatgrass Woodland |
| Utah Juniper / Basin Big sagebrush Woodland | Utah Juniper / Basin Big sagebrush Woodland |
| Utah Juniper / Bluebunch wheatgrass Woodland | Utah Juniper / Bluebunch wheatgrass Woodland |
| Utah Juniper / buckbrush / Sandburg bluegrass | Utah Juniper / buckbrush / Sandburg bluegrass |
| Utah Juniper / Indian Ricegrass Woodland | Utah Juniper / Indian Ricegrass Woodland |
| Western Wheatgrass Herbaceous | Western Wheatgrass Herbaceous |
| Winter fat / Indian Ricegrass dwarf-shrubland | Winter fat / Indian Ricegrass dwarf-shrubland |
| Yellow Wildrye Grassland | Yellow Wildrye Grassland |

### Table 6. Association Shortened Scientific Name

| AssShortSciName |  |
| --- | --- |
| Code | Description |
| Achnatherum nelsonii / Lupinus argenteus Herbaceous Vegetation | Achn nelsonii / Lupinus argenteus Herbaceous Vegetation |
| Achnatherum thurberianum - Bromus tectorum ruderal Herbaceous Vegetation | Achn thurberianum - Bromus tectorum ruderal Herbaceous Vegetation |
| Agropyron cristatum - Bromus tectorum ruderal Herbaceous Vegetation | Agro cris - Bromus tectorum ruderal Herbaceous Vegetation |
| Agropyron cristatum Western Ruderal Perennial Grassland Alliance | Agro cris Western Ruderal Perennial Grassland Alliance |
| Alnus incana / Mesic Forbs Shrubland | Alnus incana / Mesic Forbs Shrubland |
| Amaranthus californica Mudflat Herbaceous Vegetation | Amaranthus californica Mudflat Herbaceous Vegetation |
| Art trid - (Ericameria nauseosa) / Bromus tectorum Ruderal Shrubland | Art trid - (Ericameria nauseosa) / Bromus tectorum Ruderal Shrubland |
| Art trid (ssp. trid, ssp. xericensis) / Pse spi Shrub Herbaceous Vegetation | Art trid (ssp. trid, ssp. xericensis) / Pse spi Shrub Herbaceous Vegetation |
| Art trid ssp. trid / Fes ida Shrubland | Art trid ssp. trid / Fes ida Shrubland |
| Art trid ssp. trid / Hesperostipa comata Shrubland | Art trid ssp. trid / Hesperostipa comata Shrubland |
| Art trid ssp. trid / Leymus cinereus Shrubland | Art trid ssp. trid / Leymus cinereus Shrubland |
| Art trid ssp. trid / Poa secunda Shrubland | Art trid ssp. trid / Poa secunda Shrubland |
| Art trid ssp. vas - Chamaebatiaria millefolium / Penstemon deustus Shrubland | Art trid ssp. vas - Chamaebatiaria millefolium / Penstemon deustus Shrubland |
| Art trid ssp. vas - Purshia trid / Poa secunda Shrubland | Art trid ssp. vas - Purshia trid / Poa secunda Shrubland |
| Art trid ssp. vas - Purshia trid / Pse spi Shrubland | Art trid ssp. vas - Purshia trid / Pse spi Shrubland |
| Art trid ssp. vas - Symphoricarpos oreophilus / Pse spi Shrubland | Art trid ssp. vas - Symphoricarpos oreophilus / Pse spi Shrubland |
| Art trid ssp. vas / Bromus tectorum ruderal Shrubland | Art trid ssp. vas / Bromus tectorum ruderal Shrubland |
| Art trid ssp. vas / Fes ida Shrub Herbaceous Vegetation | Art trid ssp. vas / Fes ida Shrub Herbaceous Vegetation |
| Art trid ssp. vas / Hesperostipa comata Shrubland | Art trid ssp. vas / Hesperostipa comata Shrubland |
| Art trid ssp. vas / Leymus cinereus Shrubland | Art trid ssp. vas / Leymus cinereus Shrubland |
| Art trid ssp. vas / Pse spi Shrubland | Art trid ssp. vas / Pse spi Shrubland |
| Art trid ssp. wyo/ Achnatherum hymenoides Shrubland | Art trid ssp. wyo/ Achnatherum hymenoides Shrubland |
| Art trid ssp. wyo/ Achnatherum thurberianum - Bromus tectorum ruderal Shrubland | Art trid ssp. wyo/ Achnatherum thurberianum - Bromus tectorum ruderal Shrubland |
| Art trid ssp. wyo/ Agropyron cristatum ruderal Shrubland | Art trid ssp. wyo/ Agropyron cristatum ruderal Shrubland |
| Art trid ssp. wyo/ Bromus tectorum ruderal Shrubland | Art trid ssp. wyo/ Bromus tectorum ruderal Shrubland |
| Art trid ssp. wyo/ Fes ida Shrubland | Art trid ssp. wyo/ Fes ida Shrubland |
| Art trid ssp. wyo/ Hesperostipa comata Shrubland | Art trid ssp. wyo/ Hesperostipa comata Shrubland |
| Art trid ssp. wyo/ Pascopyrum smithii Shrub Herbaceous Vegetation | Art trid ssp. wyo/ Pascopyrum smithii Shrub Herbaceous Vegetation |
| Art trid ssp. wyo/ Poa secunda - Bromus tectorum ruderal Shrubland | Art trid ssp. wyo/ Poa secunda - Bromus tectorum ruderal Shrubland |
| Art trid ssp. wyo/ Pse spi Shrubland | Art trid ssp. wyo/ Pse spi Shrubland |
| Art trip ssp. trip/ Achnatherum thurberianum Shrubland | Art trip ssp. trip/ Achnatherum thurberianum Shrubland |
| Art trip ssp. trip/ Bromus tectorum ruderal Shrubland | Art trip ssp. trip/ Bromus tectorum ruderal Shrubland |
| Art trip ssp. trip/ Fes ida Shrub Herbaceous Vegetation | Art trip ssp. trip/ Fes ida Shrub Herbaceous Vegetation |
| Art trip ssp. trip/ Hesperostipa comata Shrub Herbaceous Vegetation | Art trip ssp. trip/ Hesperostipa comata Shrub Herbaceous Vegetation |
| Art trip ssp. trip/ Koeleria macrantha Shrubland | Art trip ssp. trip/ Koeleria macrantha Shrubland |
| Art trip ssp. trip/ Leymus cinereus Shrub Herbaceous Vegetation | Art trip ssp. trip/ Leymus cinereus Shrub Herbaceous Vegetation |
| Art trip ssp. trip/ Pascopyrum smithii - Bromus tectorum ruderal Shrubland | Art trip ssp. trip/ Pascopyrum smithii - Bromus tectorum ruderal Shrubland |
| Art trip ssp. trip/ Poa secunda - Bromus tectorum ruderal Shrubland | Art trip ssp. trip/ Poa secunda - Bromus tectorum ruderal Shrubland |
| Art trip ssp. trip/ Pse spi Shrub Herbaceous Vegetation | Art trip ssp. trip/ Pse spi Shrub Herbaceous Vegetation |
| Artemisia arbuscula ssp. arbuscula / Bromus tectorum ruderal Shrubland | Art arb ssp. arb / Bromus tectorum ruderal Shrubland |
| Artemisia arbuscula ssp. arbuscula / Hesperostipa comata Shrubland | Art arb ssp. arb / Hesperostipa comata Shrubland |
| Artemisia arbuscula ssp. arbuscula / Poa secunda Shrub Herbaceous Vegetation | Art arb ssp. arb / Poa secunda Shrub Herbaceous Vegetation |
| Artemisia arbuscula ssp. arbuscula / Pse spi Shrub Herbaceous Vegetation | Art arb ssp. arb / Pse spi Shrub Herbaceous Vegetation |
| Artemisia arbuscula ssp. longiloba / Fes ida Shrub Herbaceous Vegetation | Art arb ssp. longiloba / Fes ida Shrub Herbaceous Vegetation |
| Atriplex canescens - Krascheninnikovia lanata Shrubland | Atriplex canescens - Krascheninnikovia lanata Shrubland |
| Atriplex canescens / Achnatherum hymenoides Shrubland | Atriplex canescens / Achnatherum hymenoides Shrubland |
| Bromus tectorum - Taeniatherum caput-medusae Ruderal Annual Grassland Alliance | Bromus tectorum - Taeniatherum caput-medusae Ruderal Annual Grassland Alliance |
| Bromus tectorum Ruderal Herbaceous Vegetation | Bromus tectorum Ruderal Herbaceous Vegetation |
| Carex atherodes Herbaceous Vegetation | Carex atherodes Herbaceous Vegetation |
| Carex nebrascensis Herbaceous Vegetation | Carex nebrascensis Herbaceous Vegetation |
| Carex pellita Herbaceous Vegetation | Carex pellita Herbaceous Vegetation |
| Carex praegracilis Herbaceous Vegetation | Carex praegracilis Herbaceous Vegetation |
| Carex utriculata Herbaceous Vegetation | Carex utriculata Herbaceous Vegetation |
| Cercocarpus ledifolius - Purshia trid / Poa secunda Shrubland | Cercocarpus ledifolius - Purshia trid / Poa secunda Shrubland |
| Chamaebatiaria millefolium / Penstemon deustus Sparse Vegetation | Chamaebatiaria millefolium / Penstemon deustus Sparse Vegetation |
| Chry vis / Agropyron cristatum Shrubland | Chry vis / Agropyron cristatum Shrubland |
| Chry vis / Bromus tectorum ruderal Shrubland | Chry vis / Bromus tectorum ruderal Shrubland |
| Chry vis / Hesperostipa comata Shrubland | Chry vis / Hesperostipa comata Shrubland |
| Chry vis / Pascopyrum smithii Shrubland | Chry vis / Pascopyrum smithii Shrubland |
| Chry vis / Poa pratensis Ruderal Shrub Herbaceous Vegetation | Chry vis / Poa pratensis Ruderal Shrub Herbaceous Vegetation |
| Chry vis / Poa secunda - Bromus tectorum ruderal Shrubland | Chry vis / Poa secunda - Bromus tectorum ruderal Shrubland |
| Chry vis / Pse spi Shrubland | Chry vis / Pse spi Shrubland |
| Conyza canadensis - Cirsium arvense - Lactuca serriola Ruderal Herbaceous Alliance | Conyza canadensis - Cirsium arvense - Lactuca serriola Ruderal Herbaceous Alliance |
| Distichlis spicata Herbaceous Vegetation | Distichlis spicata Herbaceous Vegetation |
| Echinochloa crus-galli Ruderal Herbaceous Vegetation | Echinochloa crus-galli Ruderal Herbaceous Vegetation |
| Elaeagnus angustifolia Ruderal Woodland | Elaeagnus angustifolia Ruderal Woodland |
| Eleocharis palustris Herbaceous Vegetation | Eleocharis palustris Herbaceous Vegetation |
| Elymus lanceolatus / Lupinus argenteus Herbaceous Vegetation | Elymus lanceolatus / Lupinus argenteus Herbaceous Vegetation |
| Elymus repens Ruderal Herbaceous Vegetation | Elymus repens Ruderal Herbaceous Vegetation |
| Ericameria nana - Holodiscus dumosus / Penstemon deustus Shrubland | Eric nana - Holodiscus dumosus / Penstemon deustus Shrubland |
| Ericameria nana / Poa secunda / Penstemon deustus Sparse Vegetation | Eric nana / Poa secunda / Penstemon deustus Sparse Vegetation |
| Ericameria nauseosa / Leymus flavescens / Psoralidium lanceolatum Shrubland | Ericameria nauseosa / Leymus flavescens / Psoralidium lanceolatum Shrubland |
| Ericameria nauseosa Shrubland | Ericameria nauseosa Shrubland |
| Eriogonum ovalifolium var. depressum Dwarf-shrubland | Eriogonum ovalifolium var. depressum Dwarf-shrubland |
| Escaped or Planeted Robinia psuedoacacia | Escaped or Planeted Robinia psuedoacacia |
| Fes ida - Pse spi Herbaceous Vegetation | Fes ida - Pse spi Herbaceous Vegetation |
| Hesperostipa comata - Achnatherum hymenoides Herbaceous Vegetation | Hesp comata - Achnatherum hymenoides Herbaceous Vegetation |
| Hesperostipa comata - Poa secunda Herbaceous Vegetation | Hesp comata - Poa secunda Herbaceous Vegetation |
| Jun ost / Art trid ssp. trid / Pseudoroegnaria spicata Woodland | Jun ost / Art trid ssp. trid / Pseudoroegnaria spicata Woodland |
| Jun ost / Art trid ssp. trid Woodland | Jun ost / Art trid ssp. trid Woodland |
| Jun ost / Art trid ssp. wyo/ Hesperostipa comata Wooded Shrubland | Jun ost / Art trid ssp. Wyo / Hesperostipa comata Wooded Shrubland |
| Jun ost / Art trid ssp. wyoWoodland | Jun ost / Art trid ssp. Wyo Woodland |
| Jun ost / Fes ida Woodland | Jun ost / Fes ida Woodland |
| Jun ost / Hesperostipa comata Woodland | Jun ost / Hesperostipa comata Woodland |
| Jun ost / Pse spi Woodland | Jun ost / Pse spi Woodland |
| Jun ost / Purshia trid / Poa secunda Woodland | Jun ost / Purshia trid / Poa secunda Woodland |
| Jun sco / Art trid ssp. wyoWoodland | Jun sco / Art trid ssp. Wyo Woodland |
| Jun sco / Poa secunda / Penstemon deustus Woodland | Jun sco / Poa secunda / Penstemon deustus Woodland |
| Juncus balticus Herbaceous Vegetation | Juncus balticus Herbaceous Vegetation |
| Juniperus (ost, sco) / Art trid ssp. wyo- Chamaebatiaria millefolium Woodland | Jun (ost, sco) / Art trid ssp. Wyo - Chamaebatiaria millefolium Woodland |
| Juniperus (sco, ost) / Ericameria nana Wooded Herbaceous Vegetation | Jun (sco, ost) / Ericameria nana Wooded Herbaceous Vegetation |
| Krascheninnikovia lanata / Achnatherum hymenoides Dwarf-shrubland | Krascheninnikovia lanata / Achnatherum hymenoides Dwarf-shrubland |
| Leymus cinereus Herbaceous Vegetation | Leymus cinereus Herbaceous Vegetation |
| No Data | No Data |
| Pascopyrum smithii Herbaceous Vegetation | Pascopyrum smithii Herbaceous Vegetation |
| Philadelphus lewisii / Penstemon deustus Sparse Vegetation | Philadelphus lewisii / Penstemon deustus Sparse Vegetation |
| Phragmites australis Western North America Temperate Ruderal Herbaceous Vegetation | Phragmites australis Western North America Temperate Ruderal Herbaceous Vegetation |
| Pinus flexilis / Chamaebatiaria millefolium / Poa secunda Sparse Vegetation | Pin flex / Chamaebatiaria millefolium / Poa secunda Sparse Vegetation |
| Pinus flexilis / Purshia trid Woodland | Pin flex / Purshia trid Woodland |
| Planted Populus deltoides - Populus alba | Planted Populus deltoides - Populus alba |
| Poa secunda - Bromus tectorum ruderal Herbaceous Vegetation | Poa sec - Bromus tectorum ruderal Herbaceous Vegetation |
| Poa secunda / Leptodactylon pungens Sparse Vegetation | Poa sec / Leptodactylon pungens Sparse Vegetation |
| Poa secunda Herbaceous Vegetation | Poa sec Herbaceous Vegetation |
| Populus angustifolia / Artemisia tridentata spp. tridentata - Rhus trilobata | Pop angust / Art trid spp. tridentata - Rhus trilobata |
| Populus balsamifera ssp. trichocarpa Northern Rocky Mountain Riparian Forest & Woodland Alliance | Pop bal ssp. tri Northern Rocky Mountain Riparian Forest & Woodland Alliance |
| Potamogeton natans Herbaceous Vegetation | Potamogeton natans Herbaceous Vegetation |
| Prunus virginiana / Leymus cinereus Shrubland | Prunus virginiana / Leymus cinereus Shrubland |
| Pse spi - Balsamorhiza sagittata - Poa secunda Herbaceous Vegetation | Pse spi - Balsamorhiza sagittata - Poa secunda Herbaceous Vegetation |
| Pse spi - Melica bulbosa Herbaceous Vegetation | Pse spi - Melica bulbosa Herbaceous Vegetation |
| Pse spi - Poa secunda / Balsamorhiza sagittata Herbaceous Vegetation | Pse spi - Poa secunda / Balsamorhiza sagittata Herbaceous Vegetation |
| Pse spi - Poa secunda Herbaceous Vegetation | Pse spi - Poa secunda Herbaceous Vegetation |
| Pse spi / Crepis acuminata Herbaceous Vegetation | Pse spi / Crepis acuminata Herbaceous Vegetation |
| Purshia trid - Ericameria nana / Penstemon deustus Shrubland | Pur trid - Ericameria nana / Penstemon deustus Shrubland |
| Purshia trid - Philadelphus lewisii / Poa secunda Shrubland | Pur trid - Philadelphus lewisii / Poa secunda Shrubland |
| Purshia trid / Achnatherum thurberianum Shrubland | Pur trid / Achnatherum thurberianum Shrubland |
| Purshia trid / Leymus cinereus Shrub Herbaceous Vegetation | Pur trid / Leymus cinereus Shrub Herbaceous Vegetation |
| Purshia trid / Pse spi - Leymus cinereus Shrub Herbaceous Vegetation | Pur trid / Pse spi - Leymus cinereus Shrub Herbaceous Vegetation |
| Purshia trid / Pse spi Shrub Herbaceous Vegetation | Pur trid / Pse spi Shrub Herbaceous Vegetation |
| Rhus trilobata Intermittently Flooded Shrubland | Rhus trilobata Intermittently Flooded Shrubland |
| Ribes aureum Shrubland | Ribes aureum Shrubland |
| Rumex crispus Ruderal Herbaceous Vegetation | Rumex crispus Ruderal Herbaceous Vegetation |
| Salix (fragilis, alba) Ruderal Woodland | Salix (fragilis, alba) Ruderal Woodland |
| Salix amygdaloides Woodland | Salix amygdaloides Woodland |
| Salix exigua / Mesic Forbs Shrubland | Salix exigua / Mesic Forbs Shrubland |
| Salix exigua / Phalaris arundinacea Ruderal Shrubland | Salix exigua / Phalaris arundinacea Ruderal Shrubland |
| Salsola spp. Ruderal Herbaceous Vegetation | Salsola spp. Ruderal Herbaceous Vegetation |
| Sarcobatus vermiculatus / Achnatherum hymenoides Shrubland | Sarco vermi / Achnatherum hymenoides Shrubland |
| Sarcobatus vermiculatus / Artemisia tridentata Shrubland | Sarco vermi / Artemisia tridentata Shrubland |
| Sarcobatus vermiculatus / Ericameria nauseosa Shrubland | Sarco vermi / Ericameria nauseosa Shrubland |
| Sarcobatus vermiculatus / Leymus cinereus Shrubland | Sarco vermi / Leymus cinereus Shrubland |
| Sarcobatus vermiculatus Shrubland | Sarcobatus vermiculatus Shrubland |
| Schoenoplectus acutus Herbaceous Vegetation | Scho acutus Herbaceous Vegetation |
| Schoenoplectus americanus Western Herbaceous Vegetation | Scho americanus Western Herbaceous Vegetation |
| Schoenoplectus maritimus Herbaceous Vegetation | Scho maritimus Herbaceous Vegetation |
| Sisymbrium altissimum / Bromus tectorum ruderal Herbaceous Vegetation | Sisymbrium altissimum / Bromus tectorum ruderal Herbaceous Vegetation |
| Sporobolus airoides - Muhlenbergia asperifolia Herbaceous Vegetation | Spor air - Muhlenbergia asperifolia Herbaceous Vegetation |
| Suaeda calceoliformis Herbaceous Vegetation | Suaeda calceoliformis Herbaceous Vegetation |
| Typha (latifolia, angustifolia) Western Herbaceous Vegetation | Typha (latifolia, angustifolia) Western Herbaceous Vegetation |

### Table 7. Group Code

| GCode |  |
| --- | --- |
| Code | Descriptio |
| G246 | G246 |
| G271 | G271 |
| G272 | G272 |
| G273 | G273 |
| G302 | G302 |
| G303 | G303 |
| G310 | G310 |
| G311 | G311 |
| G506 | G506 |
| G510 | G510 |
| G521 | G521 |
| G524 | G524 |
| G525 | G525 |
| G526 | G526 |
| G531 | G531 |
| G537 | G537 |
| G538 | G538 |
| G544 | G544 |
| G600 | G600 |
| G775 | G775 |
| G797 | G797 |
| nd | nd |
| sc | sc |
| zb | zb |
| zc | zc |
| zd | zd |
| zw | zw |

### Table 8. Group Name

| GName |  |
| --- | --- |
| Code | Descriptio |
| Arid West Interior Freshwater Emergent Marsh | Arid West Interior Freshwater Emergent Marsh |
| Bare Ground | Bare Ground |
| Central Rocky Mountain Lower Montane, Foothill & Valley Grassland | Central Rocky Mountain Lower Montane, Foothill & Valley Grassland |
| Central Rocky Mountain Montane-Foothill Deciduous Shrubland | Central Rocky Mountain Montane-Foothill Deciduous Shrubland |
| Colorado Plateau - Great Basin Juniper Woodland & Savanna | Colorado Plateau - Great Basin Juniper Woodland & Savanna |
| Cultural Vegetation | Cultural Vegetation |
| Developed | Developed |
| Great Basin & Intermountain Ruderal Dry Shrubland & Grassland | Great Basin & Intermountain Ruderal Dry Shrubland & Grassland |
| Interior West Ruderal Riparian Forest & Scrub | Interior West Ruderal Riparian Forest & Scrub |
| Intermountain Dry Tall Sagebrush Shrubland & Steppe | Intermountain Dry Tall Sagebrush Shrubland & Steppe |
| Intermountain Mesic Tall Sagebrush Shrubland & Steppe | Intermountain Mesic Tall Sagebrush Shrubland & Steppe |
| Intermountain Semi-Desert Grassland | Intermountain Semi-Desert Grassland |
| Intermountain Semi-Desert Shrubland & Steppe | Intermountain Semi-Desert Shrubland & Steppe |
| Intermountain Sparsely Vegetated Dune Scrub & Grassland | Intermountain Sparsely Vegetated Dune Scrub & Grassland |
| No Data | No Data |
| North American Desert Alkaline-Saline Herbaceous Wetland & Playa | North American Desert Alkaline-Saline Herbaceous Wetland & Playa |
| North American Desert Alkaline-Saline Shrub Wetland | North American Desert Alkaline-Saline Shrub Wetland |
| Open Water | Open Water |
| Rocky Mountain & Great Basin Lowland & Foothill Riparian Shrubland | Rocky Mountain & Great Basin Lowland & Foothill Riparian Shrubland |
| Rocky Mountain & Great Basin Montane Riparian Forest | Rocky Mountain & Great Basin Montane Riparian Forest |
| Rocky Mountain Subalpine-Montane Mesic Grassland & Meadow | Rocky Mountain Subalpine-Montane Mesic Grassland & Meadow |
| See Comments | See Comments |
| Temperate Pacific Freshwater Wet Mudflat Group | Temperate Pacific Freshwater Wet Mudflat Group |
| Vancouverian & Rocky Mountain Montane Wet Meadow & Marsh | Vancouverian & Rocky Mountain Montane Wet Meadow & Marsh |
| Western Interior Riparian Forest & Woodland | Western Interior Riparian Forest & Woodland |
| Western North American Ruderal Wet Shrubland, Meadow & Marsh | Western North American Ruderal Wet Shrubland, Meadow & Marsh |
| Western North American Temperate Freshwater Aquatic Vegetation | Western North American Temperate Freshwater Aquatic Vegetation |

### Table 9. MacroGroup Code

| MGCode |  |
| --- | --- |
| Code | Descriptio |
| M026 | M026 |
| M034 | M034 |
| M036 | M036 |
| M048 | M048 |
| M073 | M073 |
| M075 | M075 |
| M082 | M082 |
| M109 | M109 |
| M168 | M168 |
| M169 | M169 |
| M171 | M171 |
| M298 | M298 |
| M301 | M301 |
| M499 | M499 |
| M888 | M888 |

### Table 10. MacroGroup Name

| MGName |  |
| --- | --- |
| Code | Descriptio |
| Arid West Interior Freshwater Emergent Marsh | Arid West Interior Freshwater Emergent Marsh |
| Central Rocky Mountain Montane-Foothill Grassland & Shrubland | Central Rocky Mountain Montane-Foothill Grassland & Shrubland |
| Great Basin & Intermountain Dry Shrubland & Grassland | Great Basin & Intermountain Dry Shrubland & Grassland |
| Great Basin & Intermountain Tall Sagebrush Shrubland & Steppe | Great Basin & Intermountain Tall Sagebrush Shrubland & Steppe |
| Interior Warm & Cool Desert Riparian Forest | Interior Warm & Cool Desert Riparian Forest |
| Interior West Ruderal Flooded & Swamp Forest | Interior West Ruderal Flooded & Swamp Forest |
| Intermountain Singleleaf Pinyon - Utah Juniper - Western Juniper Woodland | Intermountain Singleleaf Pinyon - Utah Juniper - Western Juniper Woodland |
| Rocky Mountain & Great Basin Montane Riparian Forest | Rocky Mountain & Great Basin Montane Riparian Forest |
| Rocky Mountain-Vancouverian Subalpine-High Montane Mesic Meadow | Rocky Mountain-Vancouverian Subalpine-High Montane Mesic Meadow |
| Vancouverian Lowland Wet Shrubland, Wet Meadow & Marsh | Vancouverian Lowland Wet Shrubland, Wet Meadow & Marsh |
| Warm & Cool Desert Alkali-Saline Wetland | Warm & Cool Desert Alkali-Saline Wetland |
| Western North American Cool Semi-Desert Ruderal Scrub & Grassland | Western North American Cool Semi-Desert Ruderal Scrub & Grassland |
| Western North American Freshwater Aquatic Vegetation | Western North American Freshwater Aquatic Vegetation |
| Western North American Montane-Subalpine Wet Shrubland & Wet Meadow | Western North American Montane-Subalpine Wet Shrubland & Wet Meadow |
| Western North American Ruderal Wet Shrubland, Meadow & Marsh | Western North American Ruderal Wet Shrubland, Meadow & Marsh |

### Table 11. Species Common Name

| SpeciesCommonName |  |
| --- | --- |
| Code | Description |
| alfalfa | alfalfa |
| alkali buttercup | alkali buttercup |
| alkali muhly; scratchgrass | alkali muhly; scratchgrass |
| alkali sacaton | alkali sacaton |
| American elm | American elm |
| American licorice | American licorice |
| American sloughgrass | American sloughgrass |
| analogue sedge | analogue sedge |
| Anderson's larkspur | Anderson's larkspur |
| annual bur ragweed | annual bur ragweed |
| annual rabbitsfoot grass; rabbitsfoot | annual rabbitsfoot grass; rabbitsfoot |
| antelope bitterbrush | antelope bitterbrush |
| Antelope Island skeletonplant | Antelope Island skeletonplant |
| aquatic green algae | aquatic green algae |
| arrowleaf balsamroot | arrowleaf balsamroot |
| bald spikerush | bald spikerush |
| Baltic rush | Baltic rush |
| barnyardgrass | barnyardgrass |
| basalt milkvetch | basalt milkvetch |
| basin big sagebrush | basin big sagebrush |
| basin wildrye; Great Basin wildrye | basin wildrye; Great Basin wildrye |
| bay forget-me-not | bay forget-me-not |
| beaked sedge | beaked sedge |
| bearded sprangletop | bearded sprangletop |
| beardless wildrye | beardless wildrye |
| biennial cinquefoil | biennial cinquefoil |
| biennial wormwood | biennial wormwood |
| big devils beggartick | big devils beggartick |
| bigbract verbena | bigbract verbena |
| black cottonwood | black cottonwood |
| black medick | black medick |
| black sagebrush | black sagebrush |
| Blue Mountain buckwheat | Blue Mountain buckwheat |
| blue penstemon | blue penstemon |
| bluebunch wheatgrass | bluebunch wheatgrass |
| bog yellowcress | bog yellowcress |
| Booth's evening-primrose | Booth's evening-primrose |
| boxelder | boxelder |
| broadleaf arrowhead | broadleaf arrowhead |
| broadleaf cattail; common cattail | broadleaf cattail; common cattail |
| broadleaved pepperweed | broadleaved pepperweed |
| broom snakeweed | broom snakeweed |
| bulbous bluegrass | bulbous bluegrass |
| bull thistle | bull thistle |
| bushy bird's beak | bushy bird's beak |
| bushy knotweed | bushy knotweed |
| Canada goldenrod | Canada goldenrod |
| Canada thistle | Canada thistle |
| Canadian horseweed | Canadian horseweed |
| catnip | catnip |
| chairmaker's bulrush | chairmaker's bulrush |
| cheatgrass | cheatgrass |
| chokecherry | chokecherry |
| clasping pepperweed | clasping pepperweed |
| climbing nightshade | climbing nightshade |
| cloaked bulrush | cloaked bulrush |
| clustered broomrape | clustered broomrape |
| clustered field sedge | clustered field sedge |
| common dandelion | common dandelion |
| common duckweed | common duckweed |
| common evening-primrose; hairy evening-primrose | common evening-primrose; hairy evening-primrose |
| common kochia; summercypress; burning bush | common kochia; summercypress; burning bush |
| common mallow | common mallow |
| common mullein | common mullein |
| common reed | common reed |
| common sowthistle | common sowthistle |
| common spikerush | common spikerush |
| common sunflower | common sunflower |
| common threesquare | common threesquare |
| common/gray rabbitbrush | common/gray rabbitbrush |
| coon's tail; coontail | coon's tail; coontail |
| corn | corn |
| cosmopolitan bulrush | cosmopolitan bulrush |
| crack willow | crack willow |
| creeping bentgrass | creeping bentgrass |
| creeping meadow foxtail | creeping meadow foxtail |
| crested wheatgrass | crested wheatgrass |
| crossflower | crossflower |
| Cryptantha | Cryptantha |
| curly dock | curly dock |
| curlycup gumweed | curlycup gumweed |
| curlytop knotweed | curlytop knotweed |
| cushion buckwheat | cushion buckwheat |
| cushion cryptantha | cushion cryptantha |
| deathcamas | deathcamas |
| Douglas' dustymaiden | Douglas' dustymaiden |
| Douglas' sedge | Douglas' sedge |
| Dyer's woad | Dyer's woad |
| Eaton's aster | Eaton's aster |
| entireleaved thelypody | entireleaved thelypody |
| fernleaf biscuitroot | fernleaf biscuitroot |
| field bindweed | field bindweed |
| field pennycress | field pennycress |
| fireberry hawthorn | fireberry hawthorn |
| fivehorn smotherweed | fivehorn smotherweed |
| flatspine stickseed | flatspine stickseed |
| Fleabane | Fleabane |
| flixweed; tansymustard; herb sophia | flixweed; tansymustard; herb sophia |
| floating pondweed | floating pondweed |
| fourwing saltbush | fourwing saltbush |
| fowl bluegrass | fowl bluegrass |
| fowl mannagrass | fowl mannagrass |
| fox sedge | fox sedge |
| foxtail barley | foxtail barley |
| Franklin's sandwort | Franklin's sandwort |
| freckled milkvetch | freckled milkvetch |
| Fremont's goosefoot | Fremont's goosefoot |
| fringed willowherb | fringed willowherb |
| Geranium | Geranium |
| German-madwort | German-madwort |
| giant sumpweed | giant sumpweed |
| golden currant | golden currant |
| golden dock | golden dock |
| grand collomia | grand collomia |
| granite prickly phlox | granite prickly phlox |
| greasewood | greasewood |
| Great Plains cottonwood; eastern cottonwood | Great Plains cottonwood; eastern cottonwood |
| green ash | green ash |
| green bristlegrass | green bristlegrass |
| hardstem bulrush | hardstem bulrush |
| hoary tansyaster; hoary aster | hoary tansyaster; hoary aster |
| Holboell's rockcress | Holboell's rockcress |
| hooked bristlegrass | hooked bristlegrass |
| Hooker's balsamroot | Hooker's balsamroot |
| houndstongue | houndstongue |
| Idaho fescue | Idaho fescue |
| Indian ricegrass | Indian ricegrass |
| inland saltgrass; saltgrass | inland saltgrass; saltgrass |
| intermediate wheatgrass | intermediate wheatgrass |
| iodinebush | iodinebush |
| Japanese brome | Japanese brome |
| Kentucky bluegrass | Kentucky bluegrass |
| knotsheath sedge | knotsheath sedge |
| knotweed | knotweed |
| lambsquarters | lambsquarters |
| lanceleaf goldenweed | lanceleaf goldenweed |
| leafy pondweed | leafy pondweed |
| lemon scurfpea | lemon scurfpea |
| lemonade sumac; skunkbush sumac | lemonade sumac; skunkbush sumac |
| lesser burrdock | lesser burrdock |
| lesser Indian paintbrush | lesser Indian paintbrush |
| lesser rushy milkvetch | lesser rushy milkvetch |
| lettuce | lettuce |
| little hogweed | little hogweed |
| littleleaf pussytoes | littleleaf pussytoes |
| littlepod false flax | littlepod false flax |
| lobeleaf groundsel | lobeleaf groundsel |
| longleaf groundcherry; long-leaved groundcherry | longleaf groundcherry; long-leaved groundcherry |
| low pussytoes | low pussytoes |
| Lupine | Lupine |
| mariposa lily | mariposa lily |
| marsh skullcap | marsh skullcap |
| mat muhly | mat muhly |
| meadow barley | meadow barley |
| medusahead | medusahead |
| Menzies' fiddleneck | Menzies' fiddleneck |
| Minidoka beardtongue | Minidoka beardtongue |
| mountain tansymustard | mountain tansymustard |
| narrowleaf cattail | narrowleaf cattail |
| narrowleaf cottonwood | narrowleaf cottonwood |
| narrowleaf plantain | narrowleaf plantain |
| narrowleaf willow; coyote willow | narrowleaf willow; coyote willow |
| Nebraska sedge | Nebraska sedge |
| neckweed | neckweed |
| needle-and-thread | needle-and-thread |
| Nettle-leaved Goosefoot | Nettle-leaved Goosefoot |
| nineleaf biscuitroot | nineleaf biscuitroot |
| nodding beggartick | nodding beggartick |
| northern catalpa | northern catalpa |
| northwestern Indian paintbrush | northwestern Indian paintbrush |
| Nuttall's alkaligrass | Nuttall's alkaligrass |
| owlfruit sedge | owlfruit sedge |
| pale agoseris | pale agoseris |
| paleyellow iris | paleyellow iris |
| Paradox cinquefoil | Paradox cinquefoil |
| peachleaf willow | peachleaf willow |
| Penstomen | Penstomen |
| perennial sowthistle | perennial sowthistle |
| plains evening-primrose | plains evening-primrose |
| plains pricklypear; prickly-pear cactus | plains pricklypear; prickly-pear cactus |
| poison hemlock | poison hemlock |
| Potentilla sp | Potentilla sp |
| povertyweed | povertyweed |
| Powell's amaranth | Powell's amaranth |
| prairie fleabane | prairie fleabane |
| prairie wedgescale | prairie wedgescale |
| prickly lettuce | prickly lettuce |
| prickly sowthistle | prickly sowthistle |
| prostrate knotweed | prostrate knotweed |
| prostrate pigweed | prostrate pigweed |
| purple threeawn | purple threeawn |
| Pursh seepweed | Pursh seepweed |
| quackgrass | quackgrass |
| rayless alkali aster | rayless alkali aster |
| red clover | red clover |
| red saltwort; red glasswort; red swampfire | red saltwort; red glasswort; red swampfire |
| redwool plantain; alkali plantain | redwool plantain; alkali plantain |
| reed canarygrass | reed canarygrass |
| ribseed sandmat; corrugate-seeded spurge | ribseed sandmat; corrugate-seeded spurge |
| rice cutgrass | rice cutgrass |
| rockcress | rockcress |
| Rocky Mountain beeplant | Rocky Mountain beeplant |
| Rocky Mountain goosefoot | Rocky Mountain goosefoot |
| Rocky Mountain juniper | Rocky Mountain juniper |
| Rosa spp. | Rosa spp. |
| rough bugleweed | rough bugleweed |
| rough cockleburr; common cocklebur | rough cockleburr; common cocklebur |
| rubber rabbitbrush; gray rabbitbrush | rubber rabbitbrush; gray rabbitbrush |
| Russian olive | Russian olive |
| Russian thistle; tumbleweed | Russian thistle; tumbleweed |
| sago pondweed | sago pondweed |
| saline saltbush | saline saltbush |
| saltlover | saltlover |
| sand dropseed | sand dropseed |
| sand dune wallflower | sand dune wallflower |
| Sandberg bluegrass; Sandberg's bluegrass | Sandberg bluegrass; Sandberg's bluegrass |
| scabland penstemon | scabland penstemon |
| Scotch thistle; Scotch cottonthistle | Scotch thistle; Scotch cottonthistle |
| sea milkwort | sea milkwort |
| seaside arrowgrass | seaside arrowgrass |
| shortawn foxtail | shortawn foxtail |
| shortray fleabane | shortray fleabane |
| short-rayed alkalai aster | short-rayed alkalai aster |
| showy milkweed | showy milkweed |
| Siberian elm | Siberian elm |
| silver maple | silver maple |
| silverleaf phacelia | silverleaf phacelia |
| silverweed cinquefoil | silverweed cinquefoil |
| slender buckwheat | slender buckwheat |
| smooth bromegrass | smooth bromegrass |
| smooth horsetail | smooth horsetail |
| Snake River cryptantha | Snake River cryptantha |
| softstem bulrush | softstem bulrush |
| spear saltbush | spear saltbush |
| spiny hopsage | spiny hopsage |
| spiny phlox | spiny phlox |
| spotted fritillary | spotted fritillary |
| spotted ladysthumb | spotted ladysthumb |
| squirreltail | squirreltail |
| starry false lily of the valley; starry Solomon's-seal | starry false lily of the valley; starry Solomon's-seal |
| stickywilly | stickywilly |
| stinging nettle | stinging nettle |
| strawberry clover | strawberry clover |
| stream orchid | stream orchid |
| suncup | suncup |
| swamp milkweed | swamp milkweed |
| swamp smartweed | swamp smartweed |
| sweet sagewort | sweet sagewort |
| sweetclover | sweetclover |
| tailcup lupine | tailcup lupine |
| tall annual willowherb | tall annual willowherb |
| tall fescue | tall fescue |
| tall tumblemustard; tumble mustard | tall tumblemustard; tumble mustard |
| tall wheatgrass | tall wheatgrass |
| tapertip hawksbeard | tapertip hawksbeard |
| tapertip onion | tapertip onion |
| tarragon | tarragon |
| thorn skeleton weed | thorn skeleton weed |
| threepetal bedstraw | threepetal bedstraw |
| Thurber's needlegrass | Thurber's needlegrass |
| Torrey's rush | Torrey's rush |
| tufted lovegrass; purple lovegrass | tufted lovegrass; purple lovegrass |
| tumbling saltweed | tumbling saltweed |
| turpentine wavewing | turpentine wavewing |
| Utah juniper | Utah juniper |
| Utah serviceberry | Utah serviceberry |
| veiny dock | veiny dock |
| Virginia creeper | Virginia creeper |
| Virginia creeper; thicket creeper | Virginia creeper; thicket creeper |
| water hemlock | water hemlock |
| water knotweed | water knotweed |
| water speedwell | water speedwell |
| watercress | watercress |
| western aster | western aster |
| western goldentop; western goldenrod | western goldentop; western goldenrod |
| western marsh cudweed | western marsh cudweed |
| western poison ivy | western poison ivy |
| western snowberry | western snowberry |
| western stoneseed | western stoneseed |
| western tansymustard | western tansymustard |
| Western water hemlock | Western water hemlock |
| western wheatgrass | western wheatgrass |
| western white clematis | western white clematis |
| wheat sedge | wheat sedge |
| white bryony | white bryony |
| white poplar | white poplar |
| white sand verbena | white sand verbena |
| white willow | white willow |
| whitedaisy tidytips | whitedaisy tidytips |
| white-stemmed globemallow; Munro's globemallow | white-stemmed globemallow; Munro's globemallow |
| Wilcox's woollystar | Wilcox's woollystar |
| wild cucumber | wild cucumber |
| wild mint; field mint | wild mint; field mint |
| willow dock | willow dock |
| winterfat | winterfat |
| witchgrass | witchgrass |
| Wood's rose | Wood's rose |
| woolly plantain | woolly plantain |
| woolly sedge | woolly sedge |
| Wyoming big sagebrush | Wyoming big sagebrush |
| yellow bee plant; yellow spiderflower | yellow bee plant; yellow spiderflower |
| yellow fritillary | yellow fritillary |
| yellow rabbitbrush; green rabbitbrush | yellow rabbitbrush; green rabbitbrush |
| yellow salsify | yellow salsify |
| yellow wildrye | yellow wildrye |
| yellow willow | yellow willow |

### Table 12. Species Scientific Name

| SpeciesSciName |  |
| --- | --- |
| Code | Description |
| Abronia mellifera | Abronia mellifera |
| Acer negundo var. violaceum | Acer negundo var. violaceum |
| Acer saccharinum | Acer saccharinum |
| Achnatherum hymenoides | Achnatherum hymenoides |
| Achnatherum thurberianum | Achnatherum thurberianum |
| Agoseris glauca | Agoseris glauca |
| Agropyron (=Pseudoroegneria) spicata | Agropyron (=Pseudoroegneria) spicata |
| Agropyron cristatum | Agropyron cristatum |
| Agropyron repens | Agropyron repens |
| Agrostis stolonifera | Agrostis stolonifera |
| Allenrolfea occidentalis | Allenrolfea occidentalis |
| Allium acuminatum | Allium acuminatum |
| Alopecurus aequalis | Alopecurus aequalis |
| Alopecurus arundinaceus | Alopecurus arundinaceus |
| Amaranthus albus | Amaranthus albus |
| Amaranthus powellii | Amaranthus powellii |
| Ambrosia acanthicarpa | Ambrosia acanthicarpa |
| Amelanchier utahensis | Amelanchier utahensis |
| Amsinckia menziesii | Amsinckia menziesii |
| Antennaria dimorpha | Antennaria dimorpha |
| Antennaria microphylla | Antennaria microphylla |
| Arabis holboellii | Arabis holboellii |
| Arabis sp. | Arabis sp. |
| Arctium minus | Arctium minus |
| Arenaria franklinii | Arenaria franklinii |
| Argentina anserina | Argentina anserina |
| Aristida purpurea | Aristida purpurea |
| Artemisia annua | Artemisia annua |
| Artemisia biennis | Artemisia biennis |
| Artemisia dracunculus | Artemisia dracunculus |
| Artemisia nova | Artemisia nova |
| Artemisia tridentata ssp. tridentata | Artemisia tridentata ssp. tridentata |
| Artemisia tridentata ssp. wyomingensis | Artemisia tridentata ssp. wyomingensis |
| Asclepias incarnata | Asclepias incarnata |
| Asclepias speciosa | Asclepias speciosa |
| Asperugo procumbens | Asperugo procumbens |
| Astragalus convallarius | Astragalus convallarius |
| Astragalus filipes | Astragalus filipes |
| Astragalus lentiginosus | Astragalus lentiginosus |
| Atriplex canescens | Atriplex canescens |
| Atriplex patula | Atriplex patula |
| Atriplex rosea | Atriplex rosea |
| Atriplex subspicata | Atriplex subspicata |
| Balsamorhiza hookeri | Balsamorhiza hookeri |
| Balsamorhiza sagittata | Balsamorhiza sagittata |
| Bassia hyssopifolia | Bassia hyssopifolia |
| Bassia scoparia | Bassia scoparia |
| Beckmannia syzigachne | Beckmannia syzigachne |
| Bidens cernua | Bidens cernua |
| Bidens vulgata | Bidens vulgata |
| Bolboschoenus maritimus | Bolboschoenus maritimus |
| Bromus inermis | Bromus inermis |
| Bromus japonicus | Bromus japonicus |
| Bromus tectorum | Bromus tectorum |
| Bryonia alba | Bryonia alba |
| Calochortus | Calochortus |
| Camelina microcarpa | Camelina microcarpa |
| Camissonia boothii | Camissonia boothii |
| Camissonia contorta | Camissonia contorta |
| Camissonia sp. | Camissonia sp. |
| Carex atherodes | Carex atherodes |
| Carex douglasii | Carex douglasii |
| Carex nebrascensis | Carex nebrascensis |
| Carex pellita | Carex pellita |
| Carex praegracilis | Carex praegracilis |
| Carex retrorsa | Carex retrorsa |
| Carex simulata | Carex simulata |
| Carex stipata | Carex stipata |
| Carex utriculata | Carex utriculata |
| Carex vulpinoidea | Carex vulpinoidea |
| Castilleja angustifolia | Castilleja angustifolia |
| Castilleja minor ssp. minor | Castilleja minor ssp. minor |
| Catalpa speciosa | Catalpa speciosa |
| Ceratophyllum demersum | Ceratophyllum demersum |
| Chaenactis douglasii | Chaenactis douglasii |
| Chamaesyce glyptosperma | Chamaesyce glyptosperma |
| Chara sp. | Chara sp. |
| Chenopodium album | Chenopodium album |
| Chenopodium fremontii | Chenopodium fremontii |
| Chenopodium murale | Chenopodium murale |
| Chenopodium salinum | Chenopodium salinum |
| Chorispora tenella | Chorispora tenella |
| Chrysothamnus nauseosus | Chrysothamnus nauseosus |
| Chrysothamnus viscidiflorus | Chrysothamnus viscidiflorus |
| Cicuta maculata (?) | Cicuta maculata (?) |
| Cicutata douglasii | Cicutata douglasii |
| Cirsium arvense | Cirsium arvense |
| Cirsium vulgare | Cirsium vulgare |
| Clematis ligusticifolia | Clematis ligusticifolia |
| Cleome lutea | Cleome lutea |
| Cleome serrulata | Cleome serrulata |
| Collomia grandiflora | Collomia grandiflora |
| Conium maculatum | Conium maculatum |
| Convolvulus arvensis | Convolvulus arvensis |
| Conyza canadensis | Conyza canadensis |
| Cordylanthus ramosus | Cordylanthus ramosus |
| Crataegus chrysocarpa | Crataegus chrysocarpa |
| Crepis acuminata | Crepis acuminata |
| Cryptantha circumscissa | Cryptantha circumscissa |
| Cryptantha sp. | Cryptantha sp. |
| Cryptantha spiculifera | Cryptantha spiculifera |
| Cynoglossum officinale | Cynoglossum officinale |
| Delphinium andersonii | Delphinium andersonii |
| Descurainia incana | Descurainia incana |
| Descurainia pinnata | Descurainia pinnata |
| Descurainia sophia | Descurainia sophia |
| Distichlis spicata | Distichlis spicata |
| Echinochloa crus-galli | Echinochloa crus-galli |
| Echinocystis lobata | Echinocystis lobata |
| Elaeagnus angustifolia | Elaeagnus angustifolia |
| Eleocharis erythropoda | Eleocharis erythropoda |
| Eleocharis palustris | Eleocharis palustris |
| Elymus elymoides | Elymus elymoides |
| Elymus repens | Elymus repens |
| Epilobium brachycarpum | Epilobium brachycarpum |
| Epilobium ciliatum | Epilobium ciliatum |
| Epilobium ciliatum ssp. watsonii | Epilobium ciliatum ssp. watsonii |
| Epipactis gigantea | Epipactis gigantea |
| Equisetum laevigatum | Equisetum laevigatum |
| Eragrostis pectinacea | Eragrostis pectinacea |
| Eriastrum wilcoxii | Eriastrum wilcoxii |
| Ericameria nauseosa | Ericameria nauseosa |
| Erigeron lonchophyllus | Erigeron lonchophyllus |
| Erigeron strigosus | Erigeron strigosus |
| Eriogonum microthecum | Eriogonum microthecum |
| Eriogonum ovalifolium | Eriogonum ovalifolium |
| Eriogonum sp. | Eriogonum sp. |
| Eriogonum strictum | Eriogonum strictum |
| Erysimum capitatum | Erysimum capitatum |
| Euthamia occidentalis | Euthamia occidentalis |
| Festuca idahoensis | Festuca idahoensis |
| Fraxinus pennsylvanica | Fraxinus pennsylvanica |
| Fritillaria atropurpurea | Fritillaria atropurpurea |
| Fritillaria pudica | Fritillaria pudica |
| Galium aparine | Galium aparine |
| Galium trifidum | Galium trifidum |
| Geranium | Geranium |
| Glaux maritima | Glaux maritima |
| Glyceria striata | Glyceria striata |
| Glycyrrhiza lepidota | Glycyrrhiza lepidota |
| Gnaphalium palustre | Gnaphalium palustre |
| Grayia spinosa | Grayia spinosa |
| Grindelia squarrosa | Grindelia squarrosa |
| Gutierrezia sarothrae | Gutierrezia sarothrae |
| Halogeton glomeratus | Halogeton glomeratus |
| Helianthus annuus | Helianthus annuus |
| Hesperostipa comata ssp. comata | Hesperostipa comata ssp. comata |
| Hordeum brachyantherum | Hordeum brachyantherum |
| Hordeum jubatum | Hordeum jubatum |
| Iris pseudacorus | Iris pseudacorus |
| Isatis tinctoria | Isatis tinctoria |
| Iva axillaris | Iva axillaris |
| Iva xanthifolia | Iva xanthifolia |
| Juncus balticus | Juncus balticus |
| Juncus torreyi | Juncus torreyi |
| Juniperus osteosperma | Juniperus osteosperma |
| Juniperus scopulorum | Juniperus scopulorum |
| Krascheninnikovia lanata | Krascheninnikovia lanata |
| Lactuca | Lactuca |
| Lactuca serriola | Lactuca serriola |
| Lappula occidentalis | Lappula occidentalis |
| Layia glandulosa | Layia glandulosa |
| Leersia oryzoides | Leersia oryzoides |
| Lemna minor | Lemna minor |
| Lepidium latifolium | Lepidium latifolium |
| Lepidium perfoliatum | Lepidium perfoliatum |
| Leptochloa fusca ssp. fascicularis | Leptochloa fusca ssp. fascicularis |
| Leymus cinereus | Leymus cinereus |
| Leymus flavescens | Leymus flavescens |
| Leymus triticoides | Leymus triticoides |
| Linanthus pungens | Linanthus pungens |
| Lithospermum ruderale | Lithospermum ruderale |
| Lolium arundinaceum | Lolium arundinaceum |
| Lomatium dissectum | Lomatium dissectum |
| Lomatium triternatum | Lomatium triternatum |
| Lupinus caudatus | Lupinus caudatus |
| Lupinus sp. | Lupinus sp. |
| Lycopus asper | Lycopus asper |
| Lygodesmia dianthopsis | Lygodesmia dianthopsis |
| Machaeranthera canescens | Machaeranthera canescens |
| Maianthemum stellatum | Maianthemum stellatum |
| Malva neglecta | Malva neglecta |
| Medicago lupulina | Medicago lupulina |
| Medicago sativa | Medicago sativa |
| Melilotus officinalis | Melilotus officinalis |
| Mentha arvensis | Mentha arvensis |
| Muhlenbergia asperifolia | Muhlenbergia asperifolia |
| Muhlenbergia richardsonis | Muhlenbergia richardsonis |
| Myosotis laxa | Myosotis laxa |
| Nepeta cataria | Nepeta cataria |
| Oenothera villosa | Oenothera villosa |
| Onopordum acanthium | Onopordum acanthium |
| Opuntia polyacantha | Opuntia polyacantha |
| Orobanche fasciculata | Orobanche fasciculata |
| Packera multilobata | Packera multilobata |
| Panicum capillare | Panicum capillare |
| Parthenocissus quinquefolia | Parthenocissus quinquefolia |
| Parthenocissus vitacea | Parthenocissus vitacea |
| Pascopyrum smithii | Pascopyrum smithii |
| Penstemon cyaneus | Penstemon cyaneus |
| Penstemon deustus | Penstemon deustus |
| Penstemon perpulcher | Penstemon perpulcher |
| Penstemon sp. | Penstemon sp. |
| Phacelia hastata | Phacelia hastata |
| Phalaris arundinacea | Phalaris arundinacea |
| Phlox hoodii | Phlox hoodii |
| Phragmites australis | Phragmites australis |
| Physalis longifolia | Physalis longifolia |
| Plantago eriopoda | Plantago eriopoda |
| Plantago lanceolata | Plantago lanceolata |
| Plantago patagonica | Plantago patagonica |
| Pleiacanthus spinosus | Pleiacanthus spinosus |
| Poa bulbosa | Poa bulbosa |
| Poa palustris | Poa palustris |
| Poa pratensis | Poa pratensis |
| Poa secunda | Poa secunda |
| Polygonum amphibium | Polygonum amphibium |
| Polygonum aviculare | Polygonum aviculare |
| Polygonum hydropiperoides | Polygonum hydropiperoides |
| Polygonum lapathifolium | Polygonum lapathifolium |
| Polygonum persicaria | Polygonum persicaria |
| Polygonum ramosissimum | Polygonum ramosissimum |
| Polygonum sp. | Polygonum sp. |
| Polypogon monspeliensis | Polypogon monspeliensis |
| Populus alba | Populus alba |
| Populus angustifolia | Populus angustifolia |
| Populus balsamifera ssp. trichocarpa | Populus balsamifera ssp. trichocarpa |
| Populus deltoides | Populus deltoides |
| Portulaca oleracea | Portulaca oleracea |
| Potamogeton foliosus | Potamogeton foliosus |
| Potamogeton natans | Potamogeton natans |
| Potentilla biennis | Potentilla biennis |
| Potentilla paradoxa | Potentilla paradoxa |
| Potentilla sp | Potentilla sp |
| Prunus virginiana | Prunus virginiana |
| Pseudoroegneria spicata | Pseudoroegneria spicata |
| Psoralidium lanceolatum | Psoralidium lanceolatum |
| Pteryxia terebinthina | Pteryxia terebinthina |
| Puccinellia nuttalliana | Puccinellia nuttalliana |
| Purshia tridentata | Purshia tridentata |
| Pyrrocoma lanceolata var. lanceolata | Pyrrocoma lanceolata var. lanceolata |
| Ranunculus cymbalaria | Ranunculus cymbalaria |
| Rhus trilobata | Rhus trilobata |
| Ribes aureum | Ribes aureum |
| Rorippa nasturtium-aquaticum | Rorippa nasturtium-aquaticum |
| Rorippa palustris | Rorippa palustris |
| Rosa spp. | Rosa spp. |
| Rosa woodsii | Rosa woodsii |
| Rumex crispus | Rumex crispus |
| Rumex maritimus | Rumex maritimus |
| Rumex salicifolius | Rumex salicifolius |
| Rumex venosus | Rumex venosus |
| Sagittaria latifolia | Sagittaria latifolia |
| Salicornia rubra | Salicornia rubra |
| Salix alba | Salix alba |
| Salix amygdaloides | Salix amygdaloides |
| Salix exigua | Salix exigua |
| Salix fragilis | Salix fragilis |
| Salix lutea | Salix lutea |
| Salsola kali | Salsola kali |
| Sarcobatus vermiculatus | Sarcobatus vermiculatus |
| Schoenoplectus acutus | Schoenoplectus acutus |
| Schoenoplectus americanus | Schoenoplectus americanus |
| Schoenoplectus maritimus | Schoenoplectus maritimus |
| Schoenoplectus pungens | Schoenoplectus pungens |
| Schoenoplectus tabernaemontani | Schoenoplectus tabernaemontani |
| Scirpus pallidus | Scirpus pallidus |
| Scutellaria galericulata | Scutellaria galericulata |
| Setaria verticillata | Setaria verticillata |
| Setaria viridis | Setaria viridis |
| Sisymbrium altissimum | Sisymbrium altissimum |
| Solanum dulcamara | Solanum dulcamara |
| Solidago canadensis | Solidago canadensis |
| Sonchus arvensis | Sonchus arvensis |
| Sonchus asper (?) | Sonchus asper (?) |
| Sonchus oleraceus | Sonchus oleraceus |
| Sphaeralcea munroana | Sphaeralcea munroana |
| Sphenopholis obtusata | Sphenopholis obtusata |
| Sporobolus airoides | Sporobolus airoides |
| Sporobolus cryptandrus | Sporobolus cryptandrus |
| Stuckenia pectinata | Stuckenia pectinata |
| Suaeda calceoliformis | Suaeda calceoliformis |
| Symphoricarpos occidentalis | Symphoricarpos occidentalis |
| Symphyotrichum ascendens | Symphyotrichum ascendens |
| Symphyotrichum ciliatum | Symphyotrichum ciliatum |
| Symphyotrichum eatonii | Symphyotrichum eatonii |
| Symphyotrichum frondosum | Symphyotrichum frondosum |
| Taeniatherum caput-medusae | Taeniatherum caput-medusae |
| Taraxacum officinale | Taraxacum officinale |
| Thelypodium integrifolium | Thelypodium integrifolium |
| Thinopyrum intermedium | Thinopyrum intermedium |
| Thinopyrum ponticum | Thinopyrum ponticum |
| Thlaspi arvense | Thlaspi arvense |
| Toxicodendron rydbergii | Toxicodendron rydbergii |
| Tragopogon dubius | Tragopogon dubius |
| Trifolium fragiferum | Trifolium fragiferum |
| Trifolium pratense | Trifolium pratense |
| Triglochin maritima | Triglochin maritima |
| Typha angustifolia | Typha angustifolia |
| Typha latifolia | Typha latifolia |
| Ulmus americana | Ulmus americana |
| Ulmus pumila | Ulmus pumila |
| Urtica dioica | Urtica dioica |
| Verbascum thapsus | Verbascum thapsus |
| Verbena bracteata | Verbena bracteata |
| Veronica anagallis-aquatica | Veronica anagallis-aquatica |
| Veronica peregrina | Veronica peregrina |
| Xanthium strumarium | Xanthium strumarium |
| Zea mays | Zea mays |
| Zigadenus sp. | Zigadenus sp. |

### Table 13. EIA Scores

| EIA_Scores |  |
| --- | --- |
| Code | Descriptio |
| 1 | D (1) |
| 3 | C (3) |
| 3.5 | B-C (3.5) |
| 4 | B (4) |
| 5 | A (5) |

### Table 14. Cover Classes

| Cover_2016 |  |
| --- | --- |
| Code | Description |
| 0 | 0 |
| 1 - 3 | 1 - 3 |
| 4 - 10 | 4 - 10 |
| 16 - 25 | 16 - 25 |
| 26 - 50 | 26 - 50 |
| 51 - 75 | 51 - 75 |
| 76 - 95 | 76 - 95 |
| 96 - 100 | 96 - 100 |
| No Data | No Data |

### Table 15. Height Classes

| Height_2016 |  |
| --- | --- |
| Code | Description |
| (1) 0 - 1 | (1) 0 - 1 |
| (2) 1 - 3 | (2) 1 - 3 |
| (3) 3 - 6 | (3) 3 - 6 |
| (4) 6 - 15 | (4) 6 - 15 |
| (5) 15 - 30 | (5) 15 - 30 |
| (6) 30+ | (6) 30+ |

### Table 16.  Alliance Map Labels

| AllMap |  |
| --- | --- |
| Code | Description |
| Baltic Rush | Baltic Rush |
| Big Sagebrush | Big Sagebrush |
| Bluegrass | Bluegrass |
| Bullrush | Bullrush |
| Cattail | Cattail |
| Cheatgrass | Cheatgrass |
| Cottonwood | Cottonwood |
| Crested Wheat | Crested Wheat |
| Dune Elymus | Dune Elymus |
| GB Wildrye | GB Wildrye |
| Greasewood | Greasewood |
| Green Rabbitbrush | Green Rabbitbrush |
| Horseweed | Horseweed |
| Juniper Grass | Juniper Grass |
| Juniper Shrub | Juniper Shrub |
| Native Grassland | Native Grassland |
| Native Will | Native Will |
| Pondweed | Pondweed |
| Reed | Reed |
| Ricegrass Bluebunch - Muhly Grassland | Ricegrass Bluebunch - Muhly Grassland |
| Rubber Rabbitbrush | Rubber Rabbitbrush |
| Ruderal Forest | Ruderal Forest |
| Ruderal Willow | Ruderal Willow |
| Russian Olive | Russian Olive |
| Saltgrass | Saltgrass |
| Sandberg Bluegrass | Sandberg Bluegrass |
| Sedge | Sedge |
| Snowberry | Snowberry |
| Spikerush | Spikerush |
| Sumac | Sumac |
| Thistle | Thistle |
| Willow | Willow |
| Winterfat Steppe | Winterfat Steppe |

### Table 17. Alliance Aggregate Classes

| AllAgg |  |
| --- | --- |
| Code | Description |
| Cottonwood | Cottonwood |
| Dune Elymus | Dune Elymus |
| Emergent | Emergent |
| Grassland | Grassland |
| Grassland/Emergent | Grassland/Emergent |
| Greasewood | Greasewood |
| Juniper | Juniper |
| Ruderal Forest | Ruderal Forest |
| Russian Olive | Russian Olive |
| Shrub | Shrub |
| Snowberry | Snowberry |
| Water | Water |
| Willow | Willow |

# Versioning

Version 1: June 6, 2016