WATER MONITORING SITE ANALYSIS - WY 2020

SITE NUMBER: 	357005

SITE NAME:		Blitzen River below Sodhouse Dam

FACILITY:		Malheur NWR

EQUIPMENT:

Sutron Accubar Constant Flow Bubbler Gage.  Upper and lower staff gages.  Completed Satlink 3 telemetry installation and move of equipment to external enclosure on 08/14/2019.

GAGE-HEIGHT RECORD:  (Raw)

Raw data was converted in KiDat and imported into Wiski 7.  Non submersible pressure transducer data is imported into 100.Original.  Telemetry data in imported into 102.Original.Telemetry.

COMPLETENESS OF RECORD:

The recorder provided a complete record for this period.

DATA STORAGE:

Data Retrieval 20190814.0641:  Corrupt File

File Q:\Stations_Network\357005\357005.2019\Data\357005.20190814.0641\357005_20190820_1545.csv

Field Sheet Q:\Stations_Network\357005\357005.2019\Data\357005.20190814.0641\S_357005_20190814_064100.pdf

Data Retrieval 20191021.1530:  10/24/2014(1245) – 10/21/2019(1530)

File Q:\Stations_Network\357005\357005.2020\Data\357005.20191021.1530\357005_20191021_1530.csv

Field Sheet Q:\Stations_Network\357005\357005.2020\Data\357005.20191021.1530\S_357005_20191021_133100.pdf

Data Retrieval 20200826.1600:  10/21/2019(1545) – 08/26/2010(1600)

File Q:\Stations_Network\357005\357005.2020\Data\357005.20200826.1600\357005_20200826_1600.csv

Field Sheet Q:\Stations_Network\357005\357005.2020\Data\357005.20200826.1600\S_357005_20200826_160500.pdf

Data Retrieval 20200928.1700:  Visit only

Field Sheet Q:\Stations_Network\357005\357005.2020\Data\357005.20200928.1700\S_357005_20200928_170000.pdf

GAGE-HEIGHT RECORD:  (Edited)

Edits to gage height data were applied to 130.Edited.  Drift corrections were entered to 160.DriftCorrections and were applied to 170.PublishedRecord.

DATUM AND GAGE HEIGHT CORRECTIONS:

Last levels ran on 11/05/2015, current levels ran on 10/06/2016, next levels due 10/06/2017.

Period 08/14/2019 to 10/21/2019:  Visit to download data and perform discharge measurement.  A gage height correction of 0.00 was applied on 10/21/2019.  A shift of -0.05 was applied on 10/21/2019 based on measurement 96.  Shift may be from collapsed banks during the last high flow.

Period 10/21/2019 to 08/26/2020:  Visit to download data.  A gage height correction of 0.16 was applied on 08/26/2020 when the gage was set to the upper staff.

Period 08/26/2020 to 09/28/2020:  Visit to check stage.  A gage height correction of 0.14 was applied on 09/28/2020, based on photo of lower staff, when the gage was set to the upper staff.

RATING/SHIFTS:

The base rating for this period of record is 8.1.  Shifts were applied based on discharge measurements.  The low flow control is an iron sheet piling weir.  Rating 8.1 was created to improve the mid-range of the rating 7.1.  One shifted power law according to Lauffer was used with measurements 82 through 90 to create this new base rating.  The low end remained relatively unchanged while the upper end now deviates from historical measurements and must be confirmed.  The low and mid-range portions of the rating are considered good and the upper end poor.  Rating 8.1 calculates the record starting on 10/01/2016.

A shift of -0.05 was applied to rating 8.1 on 10/21/2019 based on measurement 96.

DISCHARGE RECORD:  (Edited)

One discharge measurement (96) were performed during this period of record.  Shifts were applied as noted above.

DayMean.Edited values were not estimated during this period of record.

REMARKS:

The record for this period is considered poor.

WORKED BY: 	Kristofor Kannarr

DATE:			02/23/2020

REVIEWED BY:

DATE: