WATER MONITORING SITE ANALYSIS - WY 2021

SITE NUMBER: 	357005

SITE NAME:		Blitzen River below Sodhouse Dam

FACILITY:		Malheur NWR

EQUIPMENT:

Sutron Accubar Constant Flow Bubbler Gage.  Upper and lower staff gages.  Completed Satlink 3 telemetry installation and move of equipment to external enclosure on 08/14/2019.

GAGE-HEIGHT RECORD:  (Raw)

Raw data was converted in KiDat and imported into Wiski 7.  Non submersible pressure transducer data is imported into 100.Original.  Telemetry data in imported into 102.Original.Telemetry.

COMPLETENESS OF RECORD:

The recorder provided a complete record for this period.

DATA STORAGE:

Data Retrieval 20200928.1700:  Visit only

Field Sheet Q:\Stations_Network\357005\357005.2020\Data\357005.20200928.1700\S_357005_20200928_170000.pdf

Data Retrieval 20210415.1015:  08/26/2020(1615) – 04/15/2021(1015)

File Q:\Stations_Network\357005\357005.2021\Data\357005.20210415.1015\357005_20210415_1015.csv

Field Sheet Q:\Stations_Network\357005\357005.2021\Data\357005.20210415.1015\S_357005_20210415_101500.pdf

Data Retrieval 20210719.1600:  04/15/2021(1030) – 07/19/2021(1600)

File Q:\Stations_Network\357005\357005.2021\Data\357005.20210719.1600\357005_20210719_1600.csv

Field Sheet Q:\Stations_Network\357005\357005.2021\Data\357005.20210719.1600\S_357005_20210719_141800.pdf

Data Retrieval 20210920.1545:  07/19/2021(1615) – 09/20/2021(1545)

File Q:\Stations_Network\357005\357005.2021\Data\357005.20210920.1545\357005_20210920_1545.csv

Field Sheet Q:\Stations_Network\357005\357005.2021\Data\357005.20210920.1545\S_357005_20210920_140800.pdf

GAGE-HEIGHT RECORD:  (Edited)

Edits to gage height data were applied to 130.Edited.  Drift corrections were entered to 160.DriftCorrections and were applied to 170.PublishedRecord.

DATUM AND GAGE HEIGHT CORRECTIONS:

Last levels ran on 11/05/2015, current levels ran on 10/06/2016, next levels due 10/06/2017.

Period 08/26/2020 to 09/28/2020:  Visit to check stage.  A gage height correction of 0.14 was applied on 09/28/2020, based on photo of lower staff, when the gage was set to the upper staff.

Period 09/28/2020 to 04/15/2021:  Visit by WRB staff to download data and perform discharge measurement.  Previous shift of -0.05 from 10/21/2019 continued to 10/01/2020 due to lack of discharge measurements and no current indication of when to begin shift based on measurement 97.  A shift of 0.02 based on measurement 97 was applied on 04/15/2021.

Period 04/15/2021 to 07/19/2021:  Visit to download data and perform discharge measurement.  A gage height correction of 0.00 was applied on 07/19/2021.  A shift of 0.06 based on measurement 98 was applied on 07/19/2021.

Period 07/19/2021 to 09/20/2021:  Visit to download data and perform discharge measurement.  A gage height correction of -0.01 was applied on 09/20/2021.  A shift of 0.07 based on measurement 99 was applied on 09/20/2021.

RATING/SHIFTS:

The base rating for this period of record is 8.1.  Shifts were applied based on discharge measurements.  The low flow control is an iron sheet piling weir.  Rating 8.1 was created to improve the mid-range of the rating 7.1.  One shifted power law according to Lauffer was used with measurements 82 through 90 to create this new base rating.  The low end remained relatively unchanged while the upper end now deviates from historical measurements and must be confirmed.  The low and mid-range portions of the rating are considered good and the upper end poor.  Rating 8.1 calculates the record starting on 10/01/2016.

A shift of 0.02 was applied to rating 8.1 on 04/15/2021 based on measurement 97.

A shift of 0.06 was applied to rating 8.1 on 07/19/2021 based on measurement 98.

A shift of 0.07 was applied to rating 8.1 on 09/20/2021 based on measurement 99.

DISCHARGE RECORD:  (Edited)

Three discharge measurements (97-99) were performed during this period of record.  Shifts were applied as noted above.

DayMean.Edited values were not estimated during this period of record.

REMARKS:

The record for this period is considered poor.

WORKED BY: 	Kristofor Kannarr

DATE:			10/05/2021

REVIEWED BY:

DATE: