WATER MONITORING SITE ANALYSIS - WY 2022

SITE NUMBER: 	357005

SITE NAME:		Blitzen River below Sodhouse Dam

FACILITY:		Malheur NWR

EQUIPMENT:

Sutron Accubar Constant Flow Bubbler Gage.  Upper and lower staff gages.  Completed Satlink 3 telemetry installation and move of equipment to external enclosure on 08/14/2019.

GAGE-HEIGHT RECORD: (Raw)

Raw data was converted in KiDat and imported into Wiski 7.  Non submersible pressure transducer data is imported into 100.Original.  Telemetry data in imported into 102.Original.Telemetry.

COMPLETENESS OF RECORD:

The recorder provided a complete record for this period.

DATA STORAGE:

Data Retrieval 20220413.1215:  09/20/2021(1600) – 04/13/2022(1215)

File Q:\Stations_Network\357005\357005.2022\Data\357005.20220413.1215\357005_20220413_1215.csv

Field Sheet Q:\Stations_Network\357005\357005.2022\Data\357005.20220413.1215\S_357005_20220413_111000.pdf

Data Retrieval 20220613.1430:  04/13/2022(1230) – 06/13/2022(1430)

File Q:\Stations_Network\357005\357005.2022\Data\357005.20220613.1430\357005_20220613_1430.csv

Field Sheet Q:\Stations_Network\357005\357005.2022\Data\357005.20220613.1430\S_357005_20220613_142600.pdf

Field Sheet Q:\Stations_Network\357005\357005.2022\Data\357005.20220815.1346\S_357005_20220815_134600.pdf

Data Retrieval 20220818.1000:  06/13/2022(1430) – 08/18/2022(1000)

File Q:\Stations_Network\357005\357005.2022\Data\357005.20220818.1000\357005_20220818_1000.csv

Field Sheet Q:\Stations_Network\357005\357005.2022\Data\357005.20220818.1000\S_357005_20220818_085300.pdf

Data Retrieval 20221103.1500:  08/18/2022(1015) – 11/03/2022(1500)

File Q:\Stations_Network\357005\357005.2023\Data\357005.20221103.1500\357005_20221103_1500.csv

Field Sheet Q:\Stations_Network\357005\357005.2023\Data\357005.20221103.1500\S_357005_20221103_130600.pdf

Data Retrieval 20221208.1415:  11/03/2022(1515) – 12/08/2022(1415) Time Change

File Q:\Stations_Network\357005\357005.2023\Data\357005.20221208.1415\357005_20221208_1415.csv

Field Sheet Q:\Stations_Network\357005\357005.2023\Data\357005.20221208.1415\S_357005_20221208_140000.pdf

GAGE-HEIGHT RECORD: (Edited)

Edits to gage height data were applied to 130.Edited.  Drift corrections were entered to 160.DriftCorrections and were applied to 170.PublishedRecord.

DATUM AND GAGE HEIGHT CORRECTIONS:

Last levels ran on 11/05/2015, current levels ran on 10/06/2016, next levels due 10/06/2017.

Period 09/20/2021 to 04/13/2022:  Visit to download data and perform discharge measurement.  A gage height correction of 0.00 was applied on 04/13/2022.  A shift of 0.06 based on measurement 100 was applied on 04/13/2022.  Multiple periods from 12/28/2021 to 01/05/2022 were marked erroneous likely due to a frozen orifice line.  Multiple periods from 01/15/2022 to 01/18/2022 were copied from 100.Original to 130.Edited due to telemetry issues.

Period 04/13/2022 to 06/13/2022:  Visit to download data.  No discharge measurement.  High flow, no gage height read.

Period 06/13/2022 to 08/18/2022:  Visit to download data and perform discharge measurement.  A gage height correction of 0.00 was applied on 08/18/2022.  A shift of 0.06 was continued on 08/18/2022 based on measurement 101.

Period 08/18/2022 to 11/03/2022:  Visit to download data.  A gage height correction of 0.00 was applied on 11/03/2022.

Period 11/03/2022 to 12/08/2022:  Visit to download data and check on SatLink Telemetry.  Time change to GMT occurred during SatLink update.

RATING/SHIFTS:

The base rating for this period of record is 8.1.  Shifts were applied based on discharge measurements.  The low flow control is an iron sheet piling weir.  Rating 8.1 was created to improve the mid-range of the rating 7.1.  One shifted power law according to Lauffer was used with measurements 82 through 90 to create this new base rating.  The low end remained relatively unchanged while the upper end now deviates from historical measurements and must be confirmed.  The low and mid-range portions of the rating are considered good and the upper end poor.  Rating 8.1 calculates the record starting on 10/01/2016.

A shift of 0.06 was applied to rating 8.1 on 04/13/2022 based on measurement 100.

A shift of 0.06 was applied to rating 8.1 on 08/18/2022 based on measurement 101.

DISCHARGE RECORD:  (Edited)

Two discharge measurements (100-101) were performed during this period of record.  Shifts were applied as noted above.

DayMean.Edited values were not estimated during this period of record.

REMARKS:

The record for this period is considered poor.

WORKED BY: 	Kristofor Kannarr

DATE:			02/07/2022

REVIEWED BY:

DATE: