WATER MONITORING SITE ANALYSIS - WY 2023

SITE NUMBER: 	357005

SITE NAME:		Blitzen River below Sodhouse Dam

FACILITY:		Malheur NWR

EQUIPMENT:

Sutron Accubar Constant Flow Bubbler Gage.  Upper and lower staff gages.  Completed Satlink 3 telemetry installation and move of equipment to external enclosure on 08/14/2019.

GAGE-HEIGHT RECORD: (Raw)

Raw data was converted in KiDat and imported into Wiski 7.  Non submersible pressure transducer data is imported into 100.Original.  Telemetry data in imported into 102.Original.Telemetry.

COMPLETENESS OF RECORD:

The recorder provided a complete record for this period.

DATA STORAGE:

Data Retrieval 20221103.1500:  08/18/2022(1015) – 11/03/2022(1500)

File Q:\Stations_Network\357005\357005.2023\Data\357005.20221103.1500\357005_20221103_1500.csv

Field Sheet Q:\Stations_Network\357005\357005.2023\Data\357005.20221103.1500\S_357005_20221103_130600.pdf

Data Retrieval 20221208.1415:  11/03/2022(1515) – 12/08/2022(1415) Time change to GMT

File Q:\Stations_Network\357005\357005.2023\Data\357005.20221208.1415\357005_20221208_1415.csv

Field Sheet Q:\Stations_Network\357005\357005.2023\Data\357005.20221208.1415\S_357005_20221208_140000.pdf

Data Retrieval 20230426.1500:  12/08/2022(2230) – 04/26/2023(1500) GMT

File Q:\Stations_Network\357005\357005.2023\Data\357005.20230426.1500.GMT\357005_20230426_1500GMT.csv

Data Retrieval 20230426.2100:  04/26/2023(1515) – 04/26/2023(2100) GMT

File Q:\Stations_Network\357005\357005.2023\Data\357005.20230426.2100.GMT\357005_20230426_2100GMT.csv

Field SheetQ:\Stations_Network\357005\357005.2023\Data\357005.20230426.2100.GMT\S_357005_20230426_063500.pdf

Data Retrieval 20230427.1415:  04/26/2023(2115) – 04/27/2023(1415) GMT to PST time change

File Q:\Stations_Network\357005\357005.2023\Data\357005.20230427.1415.GMT\357005_20230427_1415GMT.csv

Field SheetQ:\Stations_Network\357005\357005.2023\Data\357005.20230427.1415.GMT\S_357005_20230427_062200.pdf

Data Retrieval 20230720.1500:  04/27/2023(0630) – 07/20/2023(1500)

File Q:\Stations_Network\357005\357005.2023\Data\357005.20230720.1500\357005_20230720_1500.csv

Field Sheet Q:\Stations_Network\357005\357005.2023\Data\357005.20230720.1500\S_357005_20230720_150000.csv

Data Retrieval 20231115.1430:  07/20/2023(1515) – 11/15/2023(1430)

File Q:\Stations_Network\357005\357005.2024\Data\357005.20231115.1430\357005_20231115_1430.csv

Field Sheet Q:\Stations_Network\357005\357005.2024\Data\357005.20231115.1430\S_357005_20231115_111400.pdf

GAGE-HEIGHT RECORD: (Edited)

Edits to gage height data were applied to 130.Edited.  Drift corrections were entered to 160.DriftCorrections and were applied to 170.PublishedRecord.

DATUM AND GAGE HEIGHT CORRECTIONS:

Last levels ran on 11/05/2015, current levels ran on 10/06/2016, next levels due 10/06/2017.

Period 08/18/2022 to 11/03/2022:  Visit to download data.  A gage height correction of 0.00 was applied on 11/03/2022.

Period 11/03/2022 to 12/08/2022:  Visit to download data and check on SatLink Telemetry.  Time change to GMT occurred during SatLink update.

Period 12/08/2022 to 04/26/2023:  Visit to download data, perform discharge measurement and diagnose telemetry.  No gage height correction applied.  A shift of 0.12 was applied on 04/26/2023 based on measurement 102.

Period 04/26/2023 to 04/27/2023:  Visit to change time from GMT to PST.

Period 04/27/2023 to 07/20/2023:  Visit to download data.  A gage height correction of -0.01 was applied on 07/20/2023.

Period 07/20/2023 to 11/15/2023:  Visit to download data and perform discharge measurement.  A gage height correction of -0.02 was applied on 11/15/2023.  A shift of 0.17 was applied on 11/15/2023 based on measurement 103.

RATING/SHIFTS:

The base rating for this period of record is 8.1.  Shifts were applied based on discharge measurements.  The low flow control is an iron sheet piling weir.  Rating 8.1 was created to improve the mid-range of the rating 7.1.  One shifted power law according to Lauffer was used with measurements 82 through 90 to create this new base rating.  The low end remained relatively unchanged while the upper end now deviates from historical measurements and must be confirmed.  The low and mid-range portions of the rating are considered good and the upper end poor.  Rating 8.1 calculates the record starting on 10/01/2016.

A shift of 0.12 was applied to rating 8.1 on 04/26/2023 based on measurement 102.

A shift of 0.17 was applied to rating 8.1 on 11/15/2023 based on measurement 103.

DISCHARGE RECORD:  (Edited)

Two discharge measurements (102-103) were performed during this period of record.  Shifts were applied as noted above.

DayMean.Edited values were not estimated during this period of record.

REMARKS:

The record for this period is considered good.

WORKED BY: 	Kristofor Kannarr

DATE:			01/25/2024

REVIEWED BY:

DATE: