WATER MONITORING SITE ANALYSIS - WY 2024 (10/01/2023-06/27/2024)

SITE NUMBER: 	357005

SITE NAME:		Blitzen River below Sodhouse Dam

FACILITY:		Malheur NWR

EQUIPMENT:

Sutron Accubar Constant Flow Bubbler Gage with a Sutron Satlink 3 GOES telemetry.  Upper and lower staff gages.

GAGE-HEIGHT RECORD: (Raw)

Raw data was converted in KiDat and imported into Wiski 7.  Non submersible pressure transducer data is imported into 100.Original.  Telemetry data in imported into 102.Original.Telemetry.

COMPLETENESS OF RECORD:

The recorder provided a complete record for this period (10/01/2023 – 06/27/2024).

DATA STORAGE:

Data Retrieval 20231115.1430:  07/20/2023(1515) – 11/15/2023(1430)

File Q:\Stations_Network\357005\357005.2024\Data\357005.20231115.1430\357005_20231115_1430.csv

Field Sheet Q:\Stations_Network\357005\357005.2024\Data\357005.20231115.1430\S_357005_20231115_111400.pdf

Data Retrieval 20240327.1445:  11/15/2023(1445) – 03/27/2024(1445)

File Q:\Stations_Network\357005\357005.2024\Data\357005.20240327.1445\357005_20240327_1445.csv

Field Sheet Q:\Stations_Network\357005\357005.2024\Data\357005.20240327.1445\S_357003_20240327_145500.pdf

Data Retrieval 20240627.1700:  03/27/2024(1500) – 06/27/2024(1700)

File Q:\Stations_Network\357005\357005.2024\Data\357005.20240627.1700\357005_20240627_1700.csv

Field Sheet Q:\Stations_Network\357005\357005.2024\Data\357005.20240627.1700\S_357005_20240627_142000.pdf

GAGE-HEIGHT RECORD: (Edited)

Edits to gage height data were applied to 130.Edited.  Drift corrections were entered to 160.DriftCorrections and were applied to 170.PublishedRecord.

DATUM AND GAGE HEIGHT CORRECTIONS:

Last levels ran on 11/05/2015, current levels ran on 10/06/2016, next levels due 10/06/2017.

Period 07/20/2023 to 11/15/2023:  Visit to download data and perform discharge measurement.  A gage height correction of -0.02 was applied on 11/15/2023.  A shift of 0.17 was applied to rating 8.1 on 11/15/2023 based on measurement 103 then a shift of 0.11 was applied to the start of rating 9.1.

Period 11/15/2023 to 03/27/2024:  Visit to download data.  Discharge measurement performed by refuge staff and High Desert Partnership with USGS at Sodhouse Lane bridge.  A gage height correction of -0.02 was continued.  A shift of 0.09 was applied on 03/27/2024 based on measurement 103a.

Period 03/27/2024 to 06/27/2024:  Visit to download data and perform discharge measurement. A gage height correction of -0.02 was continued.  A shift of -0.03 was applied on 06/27/2024.  No gage height correction applied.

RATING/SHIFTS:

The base rating for the is period of record is 9.1 Q(S) = 55.1957 * (S – 2.28342)^ 1.23113 [ft3/s]  Shifts were applied based on discharge measurements.  The low flow control is an iron sheet piling weir.  A single shifted power law according to Lauffer was used with measurements 96 through 104e from 2 to 8 feet to create rating 9.1.

A shift of 0.17 was applied to rating 8.1 on 11/15/2023 based on measurement 103.

A shift of 0.11 was applied to rating 9.1 on 11/15/2023 based on measurement 103.

A shift of 0.09 was applied to rating 9.1 on 03/27/2024 based on measurement 103a.

A shift of -0.06 was applied to rating 9.1 on 04/18/2024 based on measurement 103b.

A shift of 0.13 was applied to rating 9.1 on 05/16/2024 based on measurement 103c.

A shift of -0.10 was applied to rating 9.1 on 05/31/2024 based on measurement 103d.

A shift of -0.16 was applied to rating 9.1 on 06/21/2024 based on measurement 103e.

A shift of -.0.03 was applied to rating 9.1 on 06/27/2024 based on measurement 104.

DISCHARGE RECORD: (Edited)

Eight discharge measurements (103-104) were performed during this period of record.  Measurements 103a – 103e were performed by refuge staff.  Shifts were applied as noted above.  DayMean.Edited values were not estimated during this period of record.

REMARKS:

The record for this period is considered poor due to the large shift variance applied to rating 9.1.

WORKED BY: 	Kristofor Kannarr

DATE:			09/26/2024

REVIEWED BY:

DATE: